# 1.Safety precautions

cooperation.

1. 1 ( 1) The voltage and working frequency: please according the name plate of motor and contorlling box.

(2)Electromagnetic interference:Please stay away from high-frequency magnetic wave machines or Radio wave

transmitter etc.In order to prevent the generated electromagnetic waves from interfering with the drive device

and causing wrong action

(3)Grounded:To prevent noise interference or leakage accidents,Please do grounding works including sewing

machine, motor and control box.

1.2 If Disassemble motor and controlling box,please Turn off the power in advance.Dangerous high voltage

electricity in the electric control box, so please waiting more than  1 mins after Turn off the power, then to

disassemble the control box

1.3 When  reinstalling  the  motor,Note  that  there  is  a  certain  gap  between  the  stator  and  the  rotor,  it's

untouchable.

1.4To ensure the safety of life,Please turn off the power when repair the machine or threading needles.

1.5 This symbol indicates when the machine is installed,if made a mistake,the human body or the machine may

be damaged,Therefore, there will be this mark in the dangerous place of the machine.

This symbol indicates that there is high voltage, etc.Electrically dangerous places have this mark.

1.6This products guarantees a one-year warranty under normal working conditions and no-one error operation

# 2.Instructions

2.1 Reset

In the shutdown state, press ▲▼ at the same time to turn on, then press the "S" key to confirm to recover 2.2Monitoring mode

Press the S key for 2 seconds to enter the monitoring state, press  ▲▼ to change the monitoring serial number, the monitoring value cannot be modified. The specific monitoring interface is shown in the table below.

position

needle)

position

| Definition | Model(Jog needle) | Reserved | 5V(Sewing lights) |
| --- | --- | --- | --- |
| Foot | 6PIN | 5PIN | 4PIN |
| Definition | Ground wire(Jog | Reserved | Grligohunts)d wire(Sewing |
| Foot | 3PIN | 2PIN | 1PIN |

Note: If the above parameters do not match the actual parameters, please according the machine paremeters

Before use,  please  read  this  manual  and  lockstitch  machine's  manual  at  the  same  time,  thanks  for your

| Display No: | Item number | unit |  | Display No: | Item number | unit |
| --- | --- | --- | --- | --- | --- | --- |
| SPd | Speed | 10rp |  | HAL | HAL | / |
| CUr | Current | 0.1A |  | id | model | / |
| UdC | Voltage | V |  | vEr | version | / |
| PdL | Pedal | / |  |  |  |  |

# 3.Key Description

| Function parameter edit key |  | Enter or exit the editing of function parameters |
| --- | --- | --- |
| Parameter view save key |  | View and save the content of the selected parameter number: After selecting the parameter number, press this key to view and modify the operation, after modifying the parameter value, press this key to exit and save the parameter |
| Up selection key |  | 1.Increment key for parameter item in parameter selection area 2. Set the value increment key in the parameter content area |
| Down Select key |  | 1.Decrease key of parameter item in parameter selection area 2.Set the value decreasing key in the parameter content area |
| Slow start key |  | 1.To setting open or cancel the slow start sewing function 2.If this key and the‘down selection key’are combination to one key, press and hold for 1 second to turn on or cancel the slow start sewing function |
| Stop needle position selection key |  | 1.Switch the stop position of the needle after sewing (upper position/down position) 2.If this key and the‘up selection key’are combination to one key, press it for 1 second to switch the needle position |

4、Electrical control interface definition

| 3 2 1 | 6 5 4 |
| --- | --- |

5、Failure description

| Error code | Fault content | Reason | Inspection items and how to fix |
| --- | --- | --- | --- |
| E-01 | System voltage is too high | Brake circuit failure Incorrect voltage detection Actual voltage is too high | Whether the braking resistor is working properly Whether the system voltage detection circuit works normally Whether the incoming line voltage of the system is too high |
| E-02 | System voltage is too low | Incorrect voltage detection Actual voltage is low | Whether the system voltage detection circuit works normally Whether the incoming line voltage of the system is too low |
| E-03 | Operation box communication is poor | operation box is lost The communication data of the head | Whether the operation box device is damaged Whether the operation box program has been burned Whether the operation box plug is in good contact |
| E-05 | Pedal ID failure | Pedal recognition failure | Loose pedal joint |
| E-07 | Motor overload | Motor blocked,Motor overload | Whether the machine head or trimming mechanism is stuck Whether to sew fabrics above the specification thickness Whether the current detection signal is normal Whether the motor plug is in good contact |
| E-09,E11 | Motor signal failure | Motor positioning signal failure | Whether the needle stop substrate is installed in place Whether the motor encoder interface is in good contact |
| E- 14 | Encoder signal is abnormal | Motor power cable is damaged Motor sensor signal failure | Whether the motor interface is in good contact Whether the motor encoder interface is in good contact |
| E- 15 | Motor current is too large | The motor is running abnormally Current detection is abnormal | normally Whether the motor signal is normal Whether the system current detection circuit is working |
| E- 17 | Flip switch failure | Flip switch effective | Put down the machine head or check the flip switch |
| E-22 | OZ loop failure | OZ loop detection abnormal | Check whether the OZ circuit is normal |

Note: If the above faults cannot be eliminated according to the inspection items, please applicate technical support.

6、Parameter list

| Paramete r item | Description | Range | Initial value | Content value name description and remarks |
| --- | --- | --- | --- | --- |
| P01 | Maximum speed | 200-3700 RPM | 3700 RPM | Maximum speed setting during sewing |
| P02 | Sychronizer selection | 0-2 | 1 | "0" mean Needle up,"1" mean Needle down ,"2" mean without signal |
| P03 | Soft start switch | 0- 1 | 0 | "0" mean open, "1" mean turn off |
| P04 | Soft start Speed | 200- 1500 RPM | 400 | Soft start speed function setting |
| P05 | Number of soft start pins | 1- 15 | 1 | 1- 15 Number of soft start pins |
| P06 | Minimum speed | 200-500 RPM | 200 | Minimum speed setting during sewing |
| P07 | Upper needle position adjustment | 0-23 | 0 | machine head position adjustment |
| P08 | Down needle position adjustment | 0-23 | 12 | Position adjustment of lower needle stop |
| P09 | Automatically find the needle position when starting up | 0- 1 | 1 | "0" mean can't find upper needle position adjustment,"1" mean find upper needle position adjustment |
| P10 | mahcine head protection switch detection | 0-2 | 1 | "0" mean no detection,"1"Detect zero signal,"2" mean Detect positive signal |
| P11 | Acceleration curve adjustment | 1- 100 | 32 | Speed controller acceleration and climb slope setting |
| P15 | Needle supplement method | 0-3 | 3 | "0" mean Half stitch,"1" mean One stitch,"2" mean Continuous half stitch,"3"Continuous one stitch |
| P21 | Motor running direction | 0- 1 | 1 | "0" mean Clockwise,"1" mean Counterclockwise |
| P22 | Automatically running speed | 200-3700 RPM | 3500 RPM | Setting automatically check speed |
| P23 | Automatic running time | 1-250 | 20 | Automatic running-in time setting |
| P24 | Automatic stop time | 1-250 | 20 | Automatic running-in pause time setting |
| P25 | Continuous running test | 0- 1 | 0 | Speed continuous operation "0" mean turn off,"1" open |
| P26 | Intermittent running and testing | 0- 1 | 0 | Perform positioning cycle operation, "0" mean turn off,"1" open |
| P36 | Model selection | 1 | 1 | Model replacement |