1. Safety precautions:

Before use, please read the technical data and the mechanical manual in detail, and use it correctly.

1.1 (1) Power supply voltage and operating frequency: please follow the specifications specified by the motor.

(2) Electromagnetic wave interference: please stay away from the high frequency magnetic wave machine or radio wave transmitter, so as to avoid the generated electromagnetic wave interference of the drive device and thus the wrong action.

(3) Grounding: in order to prevent noise interference or leakage accident, please do a good job of grounding engineering (including nail buckle machine, servo motor, control box, vibration plate).

1.2 When removing the motor or bare plate, do not plug and plug; the bare plate has dangerous high voltage, so remove the bare plate only for more than 1 minute after turning off the power supply.

1.3 To ensure personal safety, please turn off the power when repairing the pin buckle machine or plugging the main board.

This symbol indicates that when a machine is installed, the human body or the machine may be damaged. So there will be a sign of the dangerous areas of the machinery.

This symbol indicates that there is high voltage, etc.

1. Safety precautions:

Before use, please read the technical data and the mechanical manual in detail, and use it correctly.

1.1 (1) Power supply voltage and operating frequency: please follow the specifications specified by the motor.

(2) Electromagnetic wave interference: please stay away from the high frequency magnetic wave machine or radio wave transmitter, so as to avoid the generated electromagnetic wave interference of the drive device and thus the wrong action.

(3) Grounding: in order to prevent noise interference or leakage accident, please do a good job of grounding engineering (including nail buckle machine, servo motor, control box, vibration plate).

1.2 When removing the motor or bare plate, do not plug and plug; the bare plate has dangerous high voltage, so remove the bare plate only for more than 1 minute after turning off the power supply.

1.3 To ensure personal safety, please turn off the power when repairing the pin buckle machine or plugging the main board.

This symbol indicates that when a machine is installed, the human body or the machine may be damaged. So there will be a sign of the dangerous areas of the machinery.

This symbol indicates that there is high voltage, etc.

| Socket number / name | foot position | definition | Socket number / name | foot position | definition |
| --- | --- | --- | --- | --- | --- |
| CN1 Spindle motor power plug-in | 1PIN | C each other | CN 11 Protection in place | 1PIN | BHDCF (protective solenoid valve) |
|  | 2PIN | B each other |  | 2PIN | 24V |
|  | 3PIN | A each other | CN 9 Protect the origin | 1PIN | 24V |
|  | 4PIN | E ARTH |  | 2PIN | BHYD (protection origin) |
| CN7 Spindle Encoder Plug | 1PIN | UP+(origin +) | C N31 Send solenoid valve to the right | 1PIN | Z SDCF (Left delivery solenoid) |
|  | 2PIN | UP-(origin-) |  | 2PIN | V DD_24V |
|  | 3PIN | PGND (high-pressure ground) | C N27 Left send solenoid valve | 1PIN | Y SDCF (Right delivery solenoid valve) |
|  | 4PIN | ENCB+ |  | 2PIN | V DD_24V |
|  | 5PIN | ENCB- | CN 33 The punching solenoid valve is inserted | 1PIN | TMDCF (push-touch solenoid valve) |
|  | 6PIN | 5V |  | 2PIN | 24V |
|  | 7PIN | ENCA+ | CN30 Blowing solenoid valve plug | 1PIN | CQDCF (blow air solenoid valve) |
|  | 8PIN | ENCA- |  | 2PIN | 24V |
| CN4 Left vibration disk plug | 1PIN | M+_STEPA | CN 28 The bottom button solenoid valve is inserted | 1PIN | CQDCF (blow air solenoid valve) |
|  | 2PIN | M-_STEPA |  | 2PIN | 24V |
| CN 6 Right vibration disk plug | 1PIN | M+_STEPA | CN25 Upper buckle solenoid valve socket | 1PIN | 24V |
|  | 2PIN | M-_STEPA |  | 2PIN | TMDW (push in place) |
| C N8 Step motor encoder insert | 1PIN | P HZ_STEP+ | C N32 Catch solenoid valve interface | 1PIN | ZKDCF (catch solenoid valve) |
|  | 2PIN | P HZ_STEP- |  | 2PIN | 24V |
|  | 3PIN | P GND | C N29 Push-mode solenoid valve interface | 1PIN | TMDCF (push-touch solenoid valve) |
|  | 4PIN | P HB_STEP+ |  | 2PIN | 24V |
|  | 5PIN | P HB_STEP+ | C N26 Top-mode solenoid valve interface | 1PIN | DMDCF (top solenoid) |
|  | 6PIN | 5V_2_PTC |  | 2PIN | 24V |
|  | 7PIN | P HA_STEP+ | C N15 Protect the solenoid valve interface | 1PIN | BHDCF (protective solenoid valve) |
|  | 8PIN | P HA_STEP- |  | 2PIN | 24V |
| J1 Stepper motor power plug-in | 1PIN | MA+_STEP1 | C N14 Infrared LED interface | 1PIN | LAMP(lamp) |
|  | 2PIN | MA-_STEP1 |  | 2PIN | 5V |
|  | 3PIN | MB+_STEP1 | CN20 Spindle zero-bit interface | 1PIN | 24V |
|  | 4PIN | MB-_STEP1 |  | 2PIN | GND (low-pressure ground) |
| CN13 Step into the burning mouth | 1PIN | P GND |  | 3PIN | ZZLW (spindle-zero position) |
|  | 2PIN | 5V _2_PTC |  | 4PIN | / |
|  | 3PIN | S WCLK | C N19 Operation box interface | 1PIN | RXD |
|  | 4PIN | S WDIO |  | 2PIN | TXD |
| C N3 Spindle burning mouth | 1PIN | 5V |  | 3PIN | 5V |
|  | 2PIN | PGND (high-pressure ground) |  | 4PIN | GND (low-pressure ground) |
|  | 3PIN | / |  | 5PIN | / |
|  | 4PIN | / |  | 6PIN | / |
|  | 5PIN | SWCLK (Burn Record) | C N21 Push the mold in place | 1PIN | 24V |
|  | 6PIN | SWDID (Burn and Record) |  | 2PIN | TMDW (push in place) |
| CN 5 Pedal port | 1PIN | GND (low-pressure ground) | C N17 Push mold origin | 1PIN | 24V |
|  | 2PIN | PEDAL (pedal) |  | 2PIN | TMYD (push origin) |
| C N10 Rotation in place | 1PIN | VDD_24V | C N16 obligate | 1PIN | V DD_24V |
|  | 2PIN | G ND |  | 2PIN | I O1 |
|  | 3PIN | X ZDW (Turn in place) | C N23 obligate | 1PIN | V DD_24V |
|  | 4PIN | / |  | 2PIN | I O2 |
| C N12 Spinning origin | 1PIN | V DD_24V | C N22 Top mold in place | 1PIN | DMDCF (top solenoid) |
|  | 2PIN | G ND |  | 2PIN | 24V |
|  | 3PIN | X ZYD (rotating origin) | C N18 Top ` module origin | 1PIN | 24V |
|  | 4PIN | / |  | 2PIN | DMYD (top die origin) |

| Socket number / name | foot position | definition | Socket number / name | foot position | definition |
| --- | --- | --- | --- | --- | --- |
| CN1 Spindle motor power plug-in | 1PIN | C each other | CN 11 Protection in place | 1PIN | BHDCF (protective solenoid valve) |
|  | 2PIN | B each other |  | 2PIN | 24V |
|  | 3PIN | A each other | CN 9 Protect the origin | 1PIN | 24V |
|  | 4PIN | E ARTH |  | 2PIN | BHYD (protection origin) |
| CN7 Spindle Encoder Plug | 1PIN | UP+(origin +) | C N31 Send solenoid valve to the right | 1PIN | Z SDCF (Left delivery solenoid) |
|  | 2PIN | UP-(origin-) |  | 2PIN | V DD_24V |
|  | 3PIN | PGND (high-pressure ground) | C N27 Left send solenoid valve | 1PIN | Y SDCF (Right delivery solenoid valve) |
|  | 4PIN | ENCB+ |  | 2PIN | V DD_24V |
|  | 5PIN | ENCB- | CN 33 The punching solenoid valve is inserted | 1PIN | TMDCF (push-touch solenoid valve) |
|  | 6PIN | 5V |  | 2PIN | 24V |
|  | 7PIN | ENCA+ | CN30 Blowing solenoid valve plug | 1PIN | CQDCF (blow air solenoid valve) |
|  | 8PIN | ENCA- |  | 2PIN | 24V |
| CN4 Left vibration disk plug | 1PIN | M+_STEPA | CN 28 The bottom button solenoid valve is inserted | 1PIN | CQDCF (blow air solenoid valve) |
|  | 2PIN | M-_STEPA |  | 2PIN | 24V |
| CN 6 Right vibration disk plug | 1PIN | M+_STEPA | CN25 Upper buckle solenoid valve socket | 1PIN | 24V |
|  | 2PIN | M-_STEPA |  | 2PIN | TMDW (push in place) |
| C N8 Step motor encoder insert | 1PIN | P HZ_STEP+ | C N32 Catch solenoid valve interface | 1PIN | ZKDCF (catch solenoid valve) |
|  | 2PIN | P HZ_STEP- |  | 2PIN | 24V |
|  | 3PIN | P GND | C N29 Push-mode solenoid valve interface | 1PIN | TMDCF (push-touch solenoid valve) |
|  | 4PIN | P HB_STEP+ |  | 2PIN | 24V |
|  | 5PIN | P HB_STEP+ | C N26 Top-mode solenoid valve interface | 1PIN | DMDCF (top solenoid) |
|  | 6PIN | 5V_2_PTC |  | 2PIN | 24V |
|  | 7PIN | P HA_STEP+ | C N15 Protect the solenoid valve interface | 1PIN | BHDCF (protective solenoid valve) |
|  | 8PIN | P HA_STEP- |  | 2PIN | 24V |
| J1 Stepper motor power plug-in | 1PIN | MA+_STEP1 | C N14 Infrared LED interface | 1PIN | LAMP(lamp) |
|  | 2PIN | MA-_STEP1 |  | 2PIN | 5V |
|  | 3PIN | MB+_STEP1 | CN20 Spindle zero-bit interface | 1PIN | 24V |
|  | 4PIN | MB-_STEP1 |  | 2PIN | GND (low-pressure ground) |
| CN13 Step into the burning mouth | 1PIN | P GND |  | 3PIN | ZZLW (spindle-zero position) |
|  | 2PIN | 5V _2_PTC |  | 4PIN | / |
|  | 3PIN | S WCLK | C N19 Operation box interface | 1PIN | RXD |
|  | 4PIN | S WDIO |  | 2PIN | TXD |
| C N3 Spindle burning mouth | 1PIN | 5V |  | 3PIN | 5V |
|  | 2PIN | PGND (high-pressure ground) |  | 4PIN | GND (low-pressure ground) |
|  | 3PIN | / |  | 5PIN | / |
|  | 4PIN | / |  | 6PIN | / |
|  | 5PIN | SWCLK (Burn Record) | C N21 Push the mold in place | 1PIN | 24V |
|  | 6PIN | SWDID (Burn and Record) |  | 2PIN | TMDW (push in place) |
| CN 5 Pedal port | 1PIN | GND (low-pressure ground) | C N17 Push mold origin | 1PIN | 24V |
|  | 2PIN | PEDAL (pedal) |  | 2PIN | TMYD (push origin) |
| C N10 Rotation in place | 1PIN | VDD_24V | C N16 obligate | 1PIN | V DD_24V |
|  | 2PIN | G ND |  | 2PIN | I O1 |
|  | 3PIN | X ZDW (Turn in place) | C N23 obligate | 1PIN | V DD_24V |
|  | 4PIN | / |  | 2PIN | I O2 |
| C N12 Spinning origin | 1PIN | V DD_24V | C N22 Top mold in place | 1PIN | DMDCF (top solenoid) |
|  | 2PIN | G ND |  | 2PIN | 24V |
|  | 3PIN | X ZYD (rotating origin) | C N18 Top ` module origin | 1PIN | 24V |
|  | 4PIN | / |  | 2PIN | DMYD (top die origin) |

Vii. Interface definition:

Vii. Interface definition:

2. Instructions for using the operation box:

| number | project | content | Set the scope | Windows default |
| --- | --- | --- | --- | --- |
| Timing adjustment of the solenoid valve |  |  |  |  |
| P-01 | The protection action closes the delay time | The protection action closes the delay time | 0～250 | 20 |
| P-02 | Protection action delay time | Protect the delayed action time | 0～250 | 0 |
| P-03 | operate time of protection | Protect overall action hold time (start to off) | 0～250 | 10s |
| P-04 | Top mode action turns off the delay time | Top mode action turns off the delay time | 0～250 | 20s |
| P-05 | Top-mode action delay time | The action time of the top mold | 0～250 | 0 |
| P-06 | Top mode action time | Top mode overall action hold time (start to off) | 0～250 | 10s |
| P-07 | Close the delay time | Close the delay time | 0～250 | 20s |
| P-08 | Push action delay time | The action time of the mold delay | 0～250 | 0 |
| P-09 | Push mode action time | Hold time (start to off) | 0～250 | 20s |
| P-10 | Close the delay time | Close the delay time | 0～250 | 0 |
| P-11 | Grab and buckle action delay time | The action time of the button delay | 0～250 | 0 |
| P-12 | Grab action time | Grasp overall action hold time (start to off) | 0～250 | 20s |
| P-13 | Turn off the delay time | Turn off the delay time | 0～250 | 0 |
| P-14 | Upper p action delay time | The action time of the button delay | 0～250 | 0 |
| P-15 | Upper button action time | Hold time (start to off) | 0～250 | 20s |
| P-16 | Close the delay time | Close the delay time | 0～250 | 0 |
| P-17 | Delay time of lower buckle action | The action time of the lower buckle delay | 0～250 | 0 |
| P-18 | Lower button action time | Lower time action hold time (start to off) | 0～250 | 20s |
| P-19 | Close the delay time | Close the delay time | 0～250 | 0 |
| P-20 | Blowing action delay time | The action time of the air blowing delay | 0～250 | 0 |
| P-21 | Blowing action time | Hold time (start to off) | 0～250 | 30s |
| functional parameter |  |  |  |  |
| P-31 | Spindle speed | Spindle speed setting | 200~4500 | 3000 |
| P -43 | Locking screen function setting | Lock screen function setting: 0. Invalid 1. valid | 0~1 | 0 |
| P-44 | Lock screen time setting | Lock screen time set: 5~240 (s) | 5-240 | 5 |
| P-45 | Restore factory parameters | Recovery factory parameters: 3: restore the save parameters set by the manufacturer 8: restore the electronic control manufacturer parameters | 0/3/8 | 0 |
| P-48 | Multiplication rate | Count of multiple rate setting 1: nail 1 count 15: nail 5 count 1 and so on | 1~20 | 1 |
| P-49 | Initial value of piece count is set | Initial value of piece count is set | 0～9950 | 2000 |
| P-50 | Count number display value control | 0: Changtable 1: Immutable | 0/1 | 0 |
| P-51 | Piece mode selection | 0: No piece 1: Automatic recount after the cycle to the set value 2: automatic recount after the cycle to the set value 3: Add the meter to the set value, the motor automatically stops 4: the meter decreases to the set value, the motor automatically stops | 0/1/2/3/4 | 1 |
| P-73 | Overall velocity percentage | Overall spike speed adjustment,> 100 acceleration and <100 slower | 1-255% | 100% |

2. Instructions for using the operation box:

| number | project | content | Set the scope | Windows default |
| --- | --- | --- | --- | --- |
| Timing adjustment of the solenoid valve |  |  |  |  |
| P-01 | The protection action closes the delay time | The protection action closes the delay time | 0～250 | 20 |
| P-02 | Protection action delay time | Protect the delayed action time | 0～250 | 0 |
| P-03 | operate time of protection | Protect overall action hold time (start to off) | 0～250 | 10s |
| P-04 | Top mode action turns off the delay time | Top mode action turns off the delay time | 0～250 | 20s |
| P-05 | Top-mode action delay time | The action time of the top mold | 0～250 | 0 |
| P-06 | Top mode action time | Top mode overall action hold time (start to off) | 0～250 | 10s |
| P-07 | Close the delay time | Close the delay time | 0～250 | 20s |
| P-08 | Push action delay time | The action time of the mold delay | 0～250 | 0 |
| P-09 | Push mode action time | Hold time (start to off) | 0～250 | 20s |
| P-10 | Close the delay time | Close the delay time | 0～250 | 0 |
| P-11 | Grab and buckle action delay time | The action time of the button delay | 0～250 | 0 |
| P-12 | Grab action time | Grasp overall action hold time (start to off) | 0～250 | 20s |
| P-13 | Turn off the delay time | Turn off the delay time | 0～250 | 0 |
| P-14 | Upper p action delay time | The action time of the button delay | 0～250 | 0 |
| P-15 | Upper button action time | Hold time (start to off) | 0～250 | 20s |
| P-16 | Close the delay time | Close the delay time | 0～250 | 0 |
| P-17 | Delay time of lower buckle action | The action time of the lower buckle delay | 0～250 | 0 |
| P-18 | Lower button action time | Lower time action hold time (start to off) | 0～250 | 20s |
| P-19 | Close the delay time | Close the delay time | 0～250 | 0 |
| P-20 | Blowing action delay time | The action time of the air blowing delay | 0～250 | 0 |
| P-21 | Blowing action time | Hold time (start to off) | 0～250 | 30s |
| functional parameter |  |  |  |  |
| P-31 | Spindle speed | Spindle speed setting | 200~4500 | 3000 |
| P -43 | Locking screen function setting | Lock screen function setting: 0. Invalid 1. valid | 0~1 | 0 |
| P-44 | Lock screen time setting | Lock screen time set: 5~240 (s) | 5-240 | 5 |
| P-45 | Restore factory parameters | Recovery factory parameters: 3: restore the save parameters set by the manufacturer 8: restore the electronic control manufacturer parameters | 0/3/8 | 0 |
| P-48 | Multiplication rate | Count of multiple rate setting 1: nail 1 count 15: nail 5 count 1 and so on | 1~20 | 1 |
| P-49 | Initial value of piece count is set | Initial value of piece count is set | 0～9950 | 2000 |
| P-50 | Count number display value control | 0: Changtable 1: Immutable | 0/1 | 0 |
| P-51 | Piece mode selection | 0: No piece 1: Automatic recount after the cycle to the set value 2: automatic recount after the cycle to the set value 3: Add the meter to the set value, the motor automatically stops 4: the meter decreases to the set value, the motor automatically stops | 0/1/2/3/4 | 1 |
| P-73 | Overall velocity percentage | Overall spike speed adjustment,> 100 acceleration and <100 slower | 1-255% | 100% |

| Socket number / name | foot position | definition | Socket number / name | foot position | definition |
| --- | --- | --- | --- | --- | --- |
| CN1 Spindle motor power plug-in | 1PIN | C· | CN 11 Protection in place | 1PIN | BHDCF (protective solenoid valve) |
|  | 2PIN | B |  | 2PIN | 24V |
|  | 3PIN | A | CN 9 Protect the origin | 1PIN | 24V |
|  | 4PIN | E ARTH |  | 2PIN | BHYD (protection origin) |
| CN7 Spindle Encoder Plug | 1PIN | UP+(origin +) | C N31 Send solenoid valve to the right |  |  |
|  | 2PIN | UP-(origin-) |  |  |  |
|  | 3PIN | PGND (high-pressure ground) | C N27 Left send solenoid valve |  |  |
|  | 4PIN | ENCB+ |  |  |  |
|  | 5PIN | ENCB- | CN 33 The punching solenoid valve is inserted | 1PIN | TMDCF (push-touch solenoid valve) |
|  | 6PIN | 5V |  | 2PIN | 24V |
|  | 7PIN | ENCA+ | CN30 Blowing solenoid valve plug | 1PIN | CQDCF (blow air solenoid valve) |
|  | 8PIN | ENCA- |  | 2PIN | 24V |
| CN4 Left vibration disk plug | 1PIN | M+_STEPA | CN 28 The bottom button solenoid valve is inserted | 1PIN | CQDCF (blow air solenoid valve) |
|  | 2PIN | M-_STEPA |  | 2PIN | 24V |
| CN 6 Right vibration disk plug | 1PIN | M+_STEPA | CN25 Upper buckle solenoid valve socket | 1PIN | 24V |
|  | 2PIN | M-_STEPA |  | 2PIN | TMDW (push in place) |
| C N8 Step motor encoder insert | 1PIN |  | C N32 Catch solenoid valve interface | 1PIN | ZKDCF (catch solenoid valve) |
|  | 2PIN |  |  | 2PIN | 24V |
|  | 3PIN |  | C N29 Push-mode solenoid valve interface | 1PIN | TMDCF (push-touch solenoid valve) |
|  | 4PIN |  |  | 2PIN | 24V |
|  | 5PIN |  | C N26 Top-mode solenoid valve interface | 1PIN | DMDCF (top solenoid) |
|  | 6PIN |  |  | 2PIN | 24V |
|  | 7PIN |  | C N15 Protect the solenoid valve interface | 1PIN | BHDCF (protective solenoid valve) |
|  | 8PIN |  |  | 2PIN | 24V |
| J1 Stepper motor power plug-in | 1PIN | MA+_STEP1 | C N14 Infrared LED interface | 1PIN | LAMP(lamp) |
|  | 2PIN | MA-_STEP1 |  | 2PIN | 5V |
|  | 3PIN | MB+_STEP1 | CN20 Spindle zero-bit interface | 1PIN | 24V |
|  | 4PIN | MB-_STEP1 |  | 2PIN | GND (low-pressure ground) |
| CN13 Step into the burning mouth | 1PIN |  |  | 3PIN | ZZLW (spindle-zero position) |
|  | 2PIN |  |  | 4PIN | / |
|  | 3PIN |  | C N19 Operation box interface | 1PIN | RXD |
|  | 4PIN |  |  | 2PIN | TXD |
| C N3 Spindle burning mouth | 1PIN | 5V |  | 3PIN | 5V |
|  | 2PIN | PGND (high-pressure ground) |  | 4PIN | GND (low-pressure ground) |
|  | 3PIN | / |  | 5PIN | / |
|  | 4PIN | / |  | 6PIN | / |
|  | 5PIN | SWCLK (Burn Record) | C N21 Push the mold in place | 1PIN | 24V |
|  | 6PIN | SWDID (Burn and Record) |  | 2PIN | TMDW (push in place) |
| CN 5 Pedal port | 1PIN | GND (low-pressure ground) | C N17 Push mold origin | 1PIN | 24V |
|  | 2PIN | PEDAL (pedal) |  | 2PIN | TMYD (push origin) |
| C N10 Rotation in place | 1PIN | 24V | C N16 obligate |  |  |
|  | 2PIN |  |  |  |  |
|  | 3PIN |  | C N23 obligate |  |  |
| C N12 Spinning origin | 1PIN |  |  |  |  |
|  | 2PIN |  | C N22 Top mold in place | 1PIN | DMDCF (top solenoid) |
|  | 3PIN |  |  | 2PIN | 24V |
|  | 4PIN |  | C N18 Top mold origin | 1PIN | 24V |
|  |  |  |  | 2PIN | DMYD (top die origin) |

| Socket number / name | foot position | definition | Socket number / name | foot position | definition |
| --- | --- | --- | --- | --- | --- |
| CN1 Spindle motor power plug-in | 1PIN | C· | CN 11 Protection in place | 1PIN | BHDCF (protective solenoid valve) |
|  | 2PIN | B |  | 2PIN | 24V |
|  | 3PIN | A | CN 9 Protect the origin | 1PIN | 24V |
|  | 4PIN | E ARTH |  | 2PIN | BHYD (protection origin) |
| CN7 Spindle Encoder Plug | 1PIN | UP+(origin +) | C N31 Send solenoid valve to the right |  |  |
|  | 2PIN | UP-(origin-) |  |  |  |
|  | 3PIN | PGND (high-pressure ground) | C N27 Left send solenoid valve |  |  |
|  | 4PIN | ENCB+ |  |  |  |
|  | 5PIN | ENCB- | CN 33 The punching solenoid valve is inserted | 1PIN | TMDCF (push-touch solenoid valve) |
|  | 6PIN | 5V |  | 2PIN | 24V |
|  | 7PIN | ENCA+ | CN30 Blowing solenoid valve plug | 1PIN | CQDCF (blow air solenoid valve) |
|  | 8PIN | ENCA- |  | 2PIN | 24V |
| CN4 Left vibration disk plug | 1PIN | M+_STEPA | CN 28 The bottom button solenoid valve is inserted | 1PIN | CQDCF (blow air solenoid valve) |
|  | 2PIN | M-_STEPA |  | 2PIN | 24V |
| CN 6 Right vibration disk plug | 1PIN | M+_STEPA | CN25 Upper buckle solenoid valve socket | 1PIN | 24V |
|  | 2PIN | M-_STEPA |  | 2PIN | TMDW (push in place) |
| C N8 Step motor encoder insert | 1PIN |  | C N32 Catch solenoid valve interface | 1PIN | ZKDCF (catch solenoid valve) |
|  | 2PIN |  |  | 2PIN | 24V |
|  | 3PIN |  | C N29 Push-mode solenoid valve interface | 1PIN | TMDCF (push-touch solenoid valve) |
|  | 4PIN |  |  | 2PIN | 24V |
|  | 5PIN |  | C N26 Top-mode solenoid valve interface | 1PIN | DMDCF (top solenoid) |
|  | 6PIN |  |  | 2PIN | 24V |
|  | 7PIN |  | C N15 Protect the solenoid valve interface | 1PIN | BHDCF (protective solenoid valve) |
|  | 8PIN |  |  | 2PIN | 24V |
| J1 Stepper motor power plug-in | 1PIN | MA+_STEP1 | C N14 Infrared LED interface | 1PIN | LAMP(lamp) |
|  | 2PIN | MA-_STEP1 |  | 2PIN | 5V |
|  | 3PIN | MB+_STEP1 | CN20 Spindle zero-bit interface | 1PIN | 24V |
|  | 4PIN | MB-_STEP1 |  | 2PIN | GND (low-pressure ground) |
| CN13 Step into the burning mouth | 1PIN |  |  | 3PIN | ZZLW (spindle-zero position) |
|  | 2PIN |  |  | 4PIN | / |
|  | 3PIN |  | C N19 Operation box interface | 1PIN | RXD |
|  | 4PIN |  |  | 2PIN | TXD |
| C N3 Spindle burning mouth | 1PIN | 5V |  | 3PIN | 5V |
|  | 2PIN | PGND (high-pressure ground) |  | 4PIN | GND (low-pressure ground) |
|  | 3PIN | / |  | 5PIN | / |
|  | 4PIN | / |  | 6PIN | / |
|  | 5PIN | SWCLK (Burn Record) | C N21 Push the mold in place | 1PIN | 24V |
|  | 6PIN | SWDID (Burn and Record) |  | 2PIN | TMDW (push in place) |
| CN 5 Pedal port | 1PIN | GND (low-pressure ground) | C N17 Push mold origin | 1PIN | 24V |
|  | 2PIN | PEDAL (pedal) |  | 2PIN | TMYD (push origin) |
| C N10 Rotation in place | 1PIN | 24V | C N16 obligate |  |  |
|  | 2PIN |  |  |  |  |
|  | 3PIN |  | C N23 obligate |  |  |
| C N12 Spinning origin | 1PIN |  |  |  |  |
|  | 2PIN |  | C N22 Top mold in place | 1PIN | DMDCF (top solenoid) |
|  | 3PIN |  |  | 2PIN | 24V |
|  | 4PIN |  | C N18 Top mold origin | 1PIN | 24V |
|  |  |  |  | 2PIN | DMYD (top die origin) |

IX. Fault code:

IX. Fault code:

| Fault display | Fault content | Possible cause of failure | Check the items and treatment |
| --- | --- | --- | --- |
| E011 E012 | Motor signal failure | Motor position sensor signal failure | Whether the motor plug touches the good motor signal detects whether the device is damaged |
| E021 E023 | Motor overload | motor stalling Motor overload | Whether the motor plug is in good contact, whether the mechanical structure is stuck and whether the current detection signal is normal |
| E101 | Hardware driver failure | Current detection is abnormal Drive-through | The system current detection circuit is working normally, and whether the drive device is damaged |
| E111 E112 | The system voltage is too high | The actual voltage is high Brake loop fault Error in voltage detection | Whether the system incoming line voltage is too high and whether the brake resistance is working normally Whether the system voltage test circuit is working normally |
| E121 E122 | The system voltage is too low | The actual voltage is low Error in voltage detection | Whether the system incoming line voltage is too low Whether the system voltage test circuit is working normally |
| E131 | Current detection circuit fault | Current detection is abnormal | Whether the system current detection circuit is working normally |
| E133 | OZ circuit failure | The OZ loop is abnormal | Whether the system OZ loop works properly |
| E134 | DBFLT hitch | The automatic resistance circuit is abnormal | Whether the automatic resistance plug is in good contact Whether the automatic resistance is damaged |
| E201 | The motor current is too large | Current detection is abnormal The motor is operating abnormally | Whether the system current detection circuit is working normally Whether the motor signal is normal |
| E211 E212 | The motor is operating abnormally | The motor is operating abnormally | Whether the motor plug is in good contact Whether the motor signal does not match |
| E301 | Poor communication of the operation box | Head operation box communication data is lost | Whether the operator box plug is in good contact Whether the operation box device is damaged |
| E501 | Zero-bit sensor fault | Sensor signal is abnormal | Whether the sensor connector is loose, and whether the sensor works properly Whether the system induction circuit works properly |
| E502 | Protection sensor fault 1 (origin signal detection abnormal) | Sensor signal is abnormal | Whether the sensor connector is loose, and whether the sensor works properly Whether the system induction circuit works properly |
| E503 | Protection sensor fault 2 (in place signal) | Sensor signal is abnormal | Whether the sensor connector is loose, and whether the sensor works properly Whether the system induction circuit works properly |
| E504 | Top mode sensor fault 1 (abnormal origin signal) Top-mode origin sensor signal is abnormal | Sensor signal is abnormal | Whether the sensor connector is loose, and whether the sensor works properly Whether the system induction circuit works properly |
| E505 | Top mode sensor fault 2 (in place signal) The top sensor signal is abnormal | Sensor signal is abnormal | Whether the sensor connector is loose, and whether the sensor works properly Whether the system induction circuit works properly |
| E506 | Push mode sensor fault 1 (abnormal origin signal) The push model origin sensor signal is abnormal | Sensor signal is abnormal | Whether the sensor connector is loose, and whether the sensor works properly Whether the system induction circuit works properly |
| E507 | Push mode sensor fault 2 (in place signal) The signal of the sensor is abnormal | Sensor signal is abnormal | Whether the sensor connector is loose, and whether the sensor works properly Whether the system induction circuit works properly |
| E508 | Reset signal sensor fault | Sensor signal is abnormal | Whether the sensor connector is loose, and whether the sensor works properly Whether the system induction circuit works properly |
| E509 | The swing arm zero position sensor failed | Sensor signal is abnormal | Whether the sensor connector is loose, and whether the sensor works properly Whether the system induction circuit works properly |
| E601 | Step into the hardware flow | Motor hardware overflow | System current detects whether the circuit is working properly and whether the drive device is damaged |
| E602 | Step motor software overcurrent / overload | Motor software overflow | System current detects whether the circuit is working properly and whether the drive device is damaged |
| E603 | Step motor current detection circuit fault | Motor current detection circuit | System current detects whether the circuit is working properly and whether the drive device is damaged |
| E604 | Step forward to find a mechanical zero fault | Initial mechanical angle of the motor | Whether the motor plug is in good contact |
| E605 | Step motor plugging | The encoder or rotor stuck at initial motor | Whether the motor plug is in good contact, and whether the machine is stuck |
| E606 | Step motor position error is too large | Motor-motor winding circuit | The system current detection circuit is working properly and whether the drive device is damaged |
| E607 | The cooling fan is not working to alarm | Heat dissipation fan connection line is loose | Whether the cooling fan plug is in good contact, whether the cooling fan is working properly and whether there is any dust blockage |
| P.oFF | Discharge display | The power is turned off | Wait for the power supply to reopen |
| EvAL | Try to protect the fault | Trial time to | Contact agents |
| P.bob | Part-time tips | An alarm arrives at the specified value | Press the "" key and remove the alarm |

Note: If the above faults cannot be eliminated according to the inspection items, please seek technical support.

| Fault display | Fault content | Possible cause of failure | Check the items and treatment |
| --- | --- | --- | --- |
| E011 E012 | Motor signal failure | Motor position sensor signal failure | Whether the motor plug touches the good motor signal detects whether the device is damaged |
| E021 E023 | Motor overload | motor stalling Motor overload | Whether the motor plug is in good contact, whether the mechanical structure is stuck and whether the current detection signal is normal |
| E101 | Hardware driver failure | Current detection is abnormal Drive-through | The system current detection circuit is working normally, and whether the drive device is damaged |
| E111 E112 | The system voltage is too high | The actual voltage is high Brake loop fault Error in voltage detection | Whether the system incoming line voltage is too high and whether the brake resistance is working normally Whether the system voltage test circuit is working normally |
| E121 E122 | The system voltage is too low | The actual voltage is low Error in voltage detection | Whether the system incoming line voltage is too low Whether the system voltage test circuit is working normally |
| E131 | Current detection circuit fault | Current detection is abnormal | Whether the system current detection circuit is working normally |
| E133 | OZ circuit failure | The OZ loop is abnormal | Whether the system OZ loop works properly |
| E134 | DBFLT hitch | The automatic resistance circuit is abnormal | Whether the automatic resistance plug is in good contact Whether the automatic resistance is damaged |
| E201 | The motor current is too large | Current detection is abnormal The motor is operating abnormally | Whether the system current detection circuit is working normally Whether the motor signal is normal |
| E211 E212 | The motor is operating abnormally | The motor is operating abnormally | Whether the motor plug is in good contact Whether the motor signal does not match |
| E301 | Poor communication of the operation box | Head operation box communication data is lost | Whether the operator box plug is in good contact Whether the operation box device is damaged |
| E501 | Zero-bit sensor fault | Sensor signal is abnormal | Whether the sensor connector is loose, and whether the sensor works properly Whether the system induction circuit works properly |
| E502 | Protection sensor fault 1 (origin signal detection abnormal) | Sensor signal is abnormal | Whether the sensor connector is loose, and whether the sensor works properly Whether the system induction circuit works properly |
| E503 | Protection sensor fault 2 (in place signal) | Sensor signal is abnormal | Whether the sensor connector is loose, and whether the sensor works properly Whether the system induction circuit works properly |
| E504 | Top mode sensor fault 1 (abnormal origin signal) Top-mode origin sensor signal is abnormal | Sensor signal is abnormal | Whether the sensor connector is loose, and whether the sensor works properly Whether the system induction circuit works properly |
| E505 | Top mode sensor fault 2 (in place signal) The top sensor signal is abnormal | Sensor signal is abnormal | Whether the sensor connector is loose, and whether the sensor works properly Whether the system induction circuit works properly |
| E506 | Push mode sensor fault 1 (abnormal origin signal) The push model origin sensor signal is abnormal | Sensor signal is abnormal | Whether the sensor connector is loose, and whether the sensor works properly Whether the system induction circuit works properly |
| E507 | Push mode sensor fault 2 (in place signal) The signal of the sensor is abnormal | Sensor signal is abnormal | Whether the sensor connector is loose, and whether the sensor works properly Whether the system induction circuit works properly |
| E508 | Reset signal sensor fault | Sensor signal is abnormal | Whether the sensor connector is loose, and whether the sensor works properly Whether the system induction circuit works properly |
| E509 | The swing arm zero position sensor failed | Sensor signal is abnormal | Whether the sensor connector is loose, and whether the sensor works properly Whether the system induction circuit works properly |
| E601 | Step into the hardware flow | Motor hardware overflow | System current detects whether the circuit is working properly and whether the drive device is damaged |
| E602 | Step motor software overcurrent / overload | Motor software overflow | System current detects whether the circuit is working properly and whether the drive device is damaged |
| E603 | Step motor current detection circuit fault | Motor current detection circuit | System current detects whether the circuit is working properly and whether the drive device is damaged |
| E604 | Step forward to find a mechanical zero fault | Initial mechanical angle of the motor | Whether the motor plug is in good contact |
| E605 | Step motor plugging | The encoder or rotor stuck at initial motor | Whether the motor plug is in good contact, and whether the machine is stuck |
| E606 | Step motor position error is too large | Motor-motor winding circuit | The system current detection circuit is working properly and whether the drive device is damaged |
| E607 | The cooling fan is not working to alarm | Heat dissipation fan connection line is loose | Whether the cooling fan plug is in good contact, whether the cooling fan is working properly and whether there is any dust blockage |
| P.oFF | Discharge display | The power is turned off | Wait for the power supply to reopen |
| EvAL | Try to protect the fault | Trial time to | Contact agents |
| P.bob | Part-time tips | An alarm arrives at the specified value | Press the "" key and remove the alarm |

Note: If the above faults cannot be eliminated according to the inspection items, please seek technical support.

·

| number | project | content | Set the scope | Windows default |
| --- | --- | --- | --- | --- |
| Timing adjustment of the solenoid valve |  |  |  |  |
| P-01 | Waiting time after the protection action | The protection action closes the delay time | 0～2.5s | 0.05s |
| P-02 | Protection action delay time | Protect the delayed action time | 0～2.5s | 0 |
| P-03 | operate time of protection | Protect overall action hold time (start to off) | 0～2.5s | 0.15s |
| P-04 | Waiting time after the top die action | Top mode action turns off the delay time | 0～2.5s | 0 |
| P-05 | Top-mode action delay time | The action time of the top mold | 0～2.5s | 0 |
| P-06 | Top mode action time | Top mode overall action hold time (start to off) | 0～2.5s | 0.3s |
| P-07 | Waiting time after the mold action | Close the delay time | 0～2.5s | 0 |
| P-08 | Push action delay time | The action time of the mold delay | 0～2.5s | 0.1s |
| P-09 | Push mode action time | Hold time (start to off) | 0～2.5s | 0.2s |
| P-10 | Waiting time after the snap action | Close the delay time | 0～2.5s | 0.1s |
| P-11 | Grab and buckle action delay time | The action time of the button delay | 0～2.5s | 0.1s |
| P-12 | Grab action time | Grasp overall action hold time (start to off) | 0～2.5s | 0.3s |
| P-19 | Waiting time after blowing the air action | Close the delay time | 0～2.5s | 0 |
| P-20 | Blowing action delay time | The action time of the air blowing delay | 0～2.5s | 0 |
| P-21 | Blowing action time | Hold time (start to off) | 0～2.5s | 0.3s |
| P-22 | Waiting time after the punching action | The punching action closes the delay time | 0～2.5s | 0 |
| P-23 | Punch action delay time | Punch action delay time | 0～2.5s | 0 |
| P-24 | Punch action time | Poverall action hold time (start to off) | 0～2.5s | 0 |
| P-25 | Waiting time after the left feeding action | Left feed off delay time | 0～2.5s | 0 |
| P-26 | Left feeding action delay time | Action time of left feed delay | 0～2.5s | 0 |
| P-27 | Left feeding movement time | Left feed overall action hold time (start to turn-off) | 0～2.5s | 0.6s |
| P-28 | Waiting time after the right feeding action | Right feed closing delay time | 0～2.5s | 0 |
| P-29 | Delay time of the right feeding action | Action time of right feed delay | 0～2.5s | 0 |
| P-30 | Right feeding action time | Right feeding overall action hold time (start to close) | 0～2.5s | 0.5s |
| functional parameter |  |  |  |  |
| P-31 | Spindle speed | Spindle speed setting | 200～4500 | 3000 |
| P -33 | Speed ratio | Turn the main shaft 8 times to buckle for a circle | 1～10 | 8 |
| P 35 | The imum minimum velocity | Set the spindle minimum speed | 200～500 | 200 |
| P-43 | Locking screen function setting | Lock screen function setting: 0. Invalid 1. valid | 0～1 | 0 |
| P-44 | Lock screen time setting | Lock screen time set: 5~240 (s) | 5～240 | 30s |
| P-45 | Restore factory parameters | Recovery factory parameters: 3: restore the save parameters set by the manufacturer 8: restore the electronic control manufacturer parameters | 0/3/8 | 0 |
| P-48 | Multiplication rate | Count of multiple rate setting 1: nail 1 count 15: nail 5 count 1 and so on | 1～20 | 1 |
| P-49 | Initial value of piece count is set | Initial value of piece count is set | 0～9950 | 2000 |
| P-50 | Count number display value control | 0: Changtable 1: Immutable | 0/1 | 0 |
| P-51 | Piece mode selection | 0: No piece 1: Automatic recount after the loop is added to the set value 2: automatic recount after cycling to the set value 3: piece to set value, the motor automatically stops 4: the meter decreases to the set value, the motor automatically stops | 0/1/2/3/4 | 1 |
| P -57 | Infrared brightness | Infrared luminance adjustment (0~100) |  | 50% |
| P-73 | Overall velocity percentage | Overall spike speed adjustment,> 100 acceleration and <100 slower | 1～255% | 150% |
| P-74 | Protect alarm time | Protect alarm time | 0～2.5s | 0.1s |
| P-75 | Push mode alarm time | Push mode alarm time | 0～2.5s | 0.1s |
| P-76 | Top mode alarm time | Top mode alarm time | 0～2.5s | 0 |
| P-78 | Step-in motor working stroke | Step-in motor working stroke | 100～4000 | 980 |
| P-80 | Reset the stroke of the stepping motor | Reset the stroke of the stepping motor | 0～1000 | 0 |
| P-89 | Step motor rotation action time | The action time for the swing arm to complete the origin to the in-place position | 1～250ms | 220 |
| P-90 | Maximum current of the stepping motor | Stepper motor, running current | 1～4A | 3A |
| P-91 | The stepping motor maintains the current | The stepping motor maintains the current | 1～4A | 1A.5 |

·

| number | project | content | Set the scope | Windows default |
| --- | --- | --- | --- | --- |
| Timing adjustment of the solenoid valve |  |  |  |  |
| P-01 | Waiting time after the protection action | The protection action closes the delay time | 0～2.5s | 0.05s |
| P-02 | Protection action delay time | Protect the delayed action time | 0～2.5s | 0 |
| P-03 | operate time of protection | Protect overall action hold time (start to off) | 0～2.5s | 0.15s |
| P-04 | Waiting time after the top die action | Top mode action turns off the delay time | 0～2.5s | 0 |
| P-05 | Top-mode action delay time | The action time of the top mold | 0～2.5s | 0 |
| P-06 | Top mode action time | Top mode overall action hold time (start to off) | 0～2.5s | 0.3s |
| P-07 | Waiting time after the mold action | Close the delay time | 0～2.5s | 0 |
| P-08 | Push action delay time | The action time of the mold delay | 0～2.5s | 0.1s |
| P-09 | Push mode action time | Hold time (start to off) | 0～2.5s | 0.2s |
| P-10 | Waiting time after the snap action | Close the delay time | 0～2.5s | 0.1s |
| P-11 | Grab and buckle action delay time | The action time of the button delay | 0～2.5s | 0.1s |
| P-12 | Grab action time | Grasp overall action hold time (start to off) | 0～2.5s | 0.3s |
| P-19 | Waiting time after blowing the air action | Close the delay time | 0～2.5s | 0 |
| P-20 | Blowing action delay time | The action time of the air blowing delay | 0～2.5s | 0 |
| P-21 | Blowing action time | Hold time (start to off) | 0～2.5s | 0.3s |
| P-22 | Waiting time after the punching action | The punching action closes the delay time | 0～2.5s | 0 |
| P-23 | Punch action delay time | Punch action delay time | 0～2.5s | 0 |
| P-24 | Punch action time | Poverall action hold time (start to off) | 0～2.5s | 0 |
| P-25 | Waiting time after the left feeding action | Left feed off delay time | 0～2.5s | 0 |
| P-26 | Left feeding action delay time | Action time of left feed delay | 0～2.5s | 0 |
| P-27 | Left feeding movement time | Left feed overall action hold time (start to turn-off) | 0～2.5s | 0.6s |
| P-28 | Waiting time after the right feeding action | Right feed closing delay time | 0～2.5s | 0 |
| P-29 | Delay time of the right feeding action | Action time of right feed delay | 0～2.5s | 0 |
| P-30 | Right feeding action time | Right feeding overall action hold time (start to close) | 0～2.5s | 0.5s |
| functional parameter |  |  |  |  |
| P-31 | Spindle speed | Spindle speed setting | 200～4500 | 3000 |
| P -33 | Speed ratio | Turn the main shaft 8 times to buckle for a circle | 1～10 | 8 |
| P 35 | The imum minimum velocity | Set the spindle minimum speed | 200～500 | 200 |
| P-43 | Locking screen function setting | Lock screen function setting: 0. Invalid 1. valid | 0～1 | 0 |
| P-44 | Lock screen time setting | Lock screen time set: 5~240 (s) | 5～240 | 30s |
| P-45 | Restore factory parameters | Recovery factory parameters: 3: restore the save parameters set by the manufacturer 8: restore the electronic control manufacturer parameters | 0/3/8 | 0 |
| P-48 | Multiplication rate | Count of multiple rate setting 1: nail 1 count 15: nail 5 count 1 and so on | 1～20 | 1 |
| P-49 | Initial value of piece count is set | Initial value of piece count is set | 0～9950 | 2000 |
| P-50 | Count number display value control | 0: Changtable 1: Immutable | 0/1 | 0 |
| P-51 | Piece mode selection | 0: No piece 1: Automatic recount after the loop is added to the set value 2: automatic recount after cycling to the set value 3: piece to set value, the motor automatically stops 4: the meter decreases to the set value, the motor automatically stops | 0/1/2/3/4 | 1 |
| P -57 | Infrared brightness | Infrared luminance adjustment (0~100) |  | 50% |
| P-73 | Overall velocity percentage | Overall spike speed adjustment,> 100 acceleration and <100 slower | 1～255% | 150% |
| P-74 | Protect alarm time | Protect alarm time | 0～2.5s | 0.1s |
| P-75 | Push mode alarm time | Push mode alarm time | 0～2.5s | 0.1s |
| P-76 | Top mode alarm time | Top mode alarm time | 0～2.5s | 0 |
| P-78 | Step-in motor working stroke | Step-in motor working stroke | 100～4000 | 980 |
| P-80 | Reset the stroke of the stepping motor | Reset the stroke of the stepping motor | 0～1000 | 0 |
| P-89 | Step motor rotation action time | The action time for the swing arm to complete the origin to the in-place position | 1～250ms | 220 |
| P-90 | Maximum current of the stepping motor | Stepper motor, running current | 1～4A | 3A |
| P-91 | The stepping motor maintains the current | The stepping motor maintains the current | 1～4A | 1A.5 |

- VIII. System parameter table:

- VIII. System parameter table:

Page 2

Page 2

Page 1

Page 1

+

+

Vi. Planning mode (addition / reduction):

Under the main interface, long press the P key to enter the parameter interface, and set the relevant parameters of P48 / P49 / P50 / P 51

P48 parameter- - - -rate setting P49 parameter- - - -target value setting P50 parameter- - - -show whether the meter value can be manually changed P51 parameter- - - -meter function switch and related mode

Display item count changes:

Under the main interface, press "" to adjust to the display section, press "" to adjust the value, and press S to confirm

Quick reset (set value):

Under the main interface, key "" zero zero (set point)

Report error: PBOB font appears on the display screen, press S key to report the error and close the recount

Vi. Planning mode (addition / reduction):

Under the main interface, long press the P key to enter the parameter interface, and set the relevant parameters of P48 / P49 / P50 / P 51

P48 parameter- - - -rate setting P49 parameter- - - -target value setting P50 parameter- - - -show whether the meter value can be manually changed P51 parameter- - - -meter function switch and related mode

Display item count changes:

Under the main interface, press "" to adjust to the display section, press "" to adjust the value, and press S to confirm

Quick reset (set value):

Under the main interface, key "" zero zero (set point)

Report error: PBOB font appears on the display screen, press S key to report the error and close the recount

5. Restore the factory Settings:

Under the main interface, long press the P key to enter the parameter interface. Entering the "P45" parameter, "0" will be displayed, then press "" to "3 or 8", and press the S key to directly restore the factory Settings.

Save the machine manufacturer level parameters 3 or 8 mode, the machine manufacturer in entering the parameter interface debugging, press "" to save the parameters, trigger will flash 3 times, that is, save complete, press P key to exit and enter the normal mode.

| P-45 | Restore factory parameters | Restore factory parameters 3: restore the saving parameters set by the manufacturer 8: restore the parameters of the electronic control manufacturer | 0/3/8 (mode selection) | 0 (Windows default) |
| --- | --- | --- | --- | --- |

5. Restore the factory Settings:

Under the main interface, long press the P key to enter the parameter interface. Entering the "P45" parameter, "0" will be displayed, then press "" to "3 or 8", and press the S key to directly restore the factory Settings.

Save the machine manufacturer level parameters 3 or 8 mode, the machine manufacturer in entering the parameter interface debugging, press "" to save the parameters, trigger will flash 3 times, that is, save complete, press P key to exit and enter the normal mode.

| P-45 | Restore factory parameters | Restore factory parameters 3: restore the saving parameters set by the manufacturer 8: restore the parameters of the electronic control manufacturer | 0/3/8 (mode selection) | 0 (Windows default) |
| --- | --- | --- | --- | --- |

+

+

+

+

four. Aging mode:

Under the main interface, press the "" key to enter the aging mode, the light is in the aging mode, press "" to start the operation, press again to stop

. To return from this mode to normal mode, press the "" key for more than 1 second.

four. Aging mode:

Under the main interface, press the "" key to enter the aging mode, the light is in the aging mode, press "" to start the operation, press again to stop

. To return from this mode to normal mode, press the "" key for more than 1 second.

three. System monitoring status:

Under the main interface, long press "" to enter the monitoring mode. Press the key to display relevant monitoring parameters and press "" to return to sewing mode.

| nominate | project name | unit |  | nominate | project name | unit |
| --- | --- | --- | --- | --- | --- | --- |
| U1 | motor speed | rpm |  | U2 | Motor current | 0.01A |
| U3 | busbar voltage | V |  | U4 | Pedal voltage | 0.01V |
| U5 | Machine head mechanical Angle | linear measure |  | U6 | Initial Angle of motor | linear measure |
| U8 | Main control version number | V xxx |  | VER | HMI, version number | V xxx |
| U9 | Main control model | S 220 |  | TYPE | HMI model | A xxx |

three. System monitoring status:

Under the main interface, long press "" to enter the monitoring mode. Press the key to display relevant monitoring parameters and press "" to return to sewing mode.

| nominate | project name | unit |  | nominate | project name | unit |
| --- | --- | --- | --- | --- | --- | --- |
| U1 | motor speed | rpm |  | U2 | Motor current | 0.01A |
| U3 | busbar voltage | V |  | U4 | Pedal voltage | 0.01V |
| U5 | Machine head mechanical Angle | linear measure |  | U6 | Initial Angle of motor | linear measure |
| U8 | Main control version number | V xxx |  | VER | HMI, version number | V xxx |
| U9 | Main control model | S 220 |  | TYPE | HMI model | A xxx |

| key | function | description |
| --- | --- | --- |
| Parameter setting key |  | In working mode or debugging mode, long press P to enter [password input interface], and display the correct password (password: 1111). |
| Determine the key |  | Confirm key; long press S key to enter [system monitoring interface]. |
| Add key \ subtract key |  | In working mode, the speed regulation effect (pressing "+, -" can directly adjust the P73-overall speed percentage) Show the segment value addition and subtraction |
| Left and right keys |  | Display several segments left and right switch In debugging mode, the vibration disc frequency / vibration disc speed is switched In working mode, short press "> or <" to flash, press + key to increase the value, and press-key to decrease the value. |
| Action track mode button |  | Click on the cycle: mode 0, mode 1, mode 2, mode 3 In the running state, the mode switching operation is invalid, and the mode can only be changed before the whole mode is completed. |
| Workplace reset key |  | Short press: in the action track mode, the solenoid valve action causes the mechanism to return to the origin working state, and the spindle motor and stepper motor look for the working origin. |
| Singles / consecutive play mode keys |  | Click cycle: single / continuous play (valid) Single: pedal pedal, the overall action cycle once Continuous beat: the pedal pedal, the overall action is circulating, if you need to pause the pedal Long press in debugging mode: continuous play pause time shortcut parameter setting P72 |
| Work / debug status button |  | Click-press cycle: working status / debugging state, and the count value is not displayed under the debugging interface. |
| Fill the buckle |  | Working status [mode 1, Mode 2, mode 3] is valid Click 1, and buckle 1 time |
| Fill the buckle |  | Working status [Mode 3] is valid Click 1, and buckle 1 |
| Push the mold |  | Effective in the debugging state Click the push button (open and close), the display icon lights up, the solenoid valve moves once, press the solenoid valve moves once again, and the display icon closes Long press: Quick interface adjustment: P7 / P8 / P9 |
| punching press |  | Effective in the debugging state Click on the stamping key to display the icon off Click the stamping + key, stamping forward running an Angle, long press + key, stamping forward slow rotation running Click the stamping-key, stamping reverse run an Angle, long press-key, stamping reverse slow rotation run |
| protect |  | Effective in the debugging state Click the protection key (open and close), the display icon lights up, the solenoid valve action once, then press the solenoid valve action once, the display icon is closed Long press: protection parameters of Quick interface adjustment: P1 / P2 / P3 |
| backform |  | Effective in the debugging state Click: Top mode key (open and close), light up the display icon, move the solenoid valve once, press the solenoid valve again once, and turn the display icon off Long press: top mode parameter quick interface adjustment: P4 / P5 / P6 |
| Left to send |  | Effective in the debugging state Click: Left send key (open and off), display icon light up and close, left send solenoid valve action once Long press: left send parameter shortcut interface adjustment: P 27 \ P26 \ P25 |
| Right to send |  | Effective in the debugging state Click the right key (open and close), display icon light and close, the right solenoid valve action once Long press: Right delivery parameter shortcut interface adjustment: P 30 \ P29 \ P28 |
| Grasp the buckle |  | Effective in the debugging state Click the button (open and close), display icon light and close, grasp the solenoid valve once Long press: quick interface adjustment: P10 / P11 / P12 |
| scavenging |  | Effective in the debugging state Click the blow key (on and off), display icon light and close, blow solenoid valve action once Long press: Quick interface adjustment: P19 / P20 / P21 |
| pilot lamp |  | Effective in the debugging state Click the indicator light key, display icon light, then click off, when the icon light press + or-automatically jump to P 57 parameter, can control the infrared brightness by increasing or reducing the parameter value of P 57. |
| Left material plate |  | Effective in the debugging state Click the left plate key (on and off) for the display icon Point / long press + key: add or subtract the value Point / long press-key: value Press left and right keys: vibration disc frequency / vibration disc speed switch (display) |
| The right plate |  | Effective in the debugging state Click the right plate key (on and off) for the display icon Point / long press + key: add or subtract the value Point / long press-key: value Press left and right keys: vibration disc frequency / vibration disc speed switch (display) |
| piercing |  | Effective in the debugging state Click the punch (open and close), display icon light, punch solenoid valve suction conduction; Long press: Quick adjustment interface: P 24 \ P23 \ P22 |
| revolve |  | Effective in the debugging state Press the rotation key (on and off), the display icon lights up, and the stepping motor rotates by 90 degrees Long press: Rotation parameters quick adjustment interface: P 89 |

| key | function | description |
| --- | --- | --- |
| Parameter setting key |  | In working mode or debugging mode, long press P to enter [password input interface], and display the correct password (password: 1111). |
| Determine the key |  | Confirm key; long press S key to enter [system monitoring interface]. |
| Add key \ subtract key |  | In working mode, the speed regulation effect (pressing "+, -" can directly adjust the P73-overall speed percentage) Show the segment value addition and subtraction |
| Left and right keys |  | Display several segments left and right switch In debugging mode, the vibration disc frequency / vibration disc speed is switched In working mode, short press "> or <" to flash, press + key to increase the value, and press-key to decrease the value. |
| Action track mode button |  | Click on the cycle: mode 0, mode 1, mode 2, mode 3 In the running state, the mode switching operation is invalid, and the mode can only be changed before the whole mode is completed. |
| Workplace reset key |  | Short press: in the action track mode, the solenoid valve action causes the mechanism to return to the origin working state, and the spindle motor and stepper motor look for the working origin. |
| Singles / consecutive play mode keys |  | Click cycle: single / continuous play (valid) Single: pedal pedal, the overall action cycle once Continuous beat: the pedal pedal, the overall action is circulating, if you need to pause the pedal Long press in debugging mode: continuous play pause time shortcut parameter setting P72 |
| Work / debug status button |  | Click-press cycle: working status / debugging state, and the count value is not displayed under the debugging interface. |
| Fill the buckle |  | Working status [mode 1, Mode 2, mode 3] is valid Click 1, and buckle 1 time |
| Fill the buckle |  | Working status [Mode 3] is valid Click 1, and buckle 1 |
| Push the mold |  | Effective in the debugging state Click the push button (open and close), the display icon lights up, the solenoid valve moves once, press the solenoid valve moves once again, and the display icon closes Long press: Quick interface adjustment: P7 / P8 / P9 |
| punching press |  | Effective in the debugging state Click on the stamping key to display the icon off Click the stamping + key, stamping forward running an Angle, long press + key, stamping forward slow rotation running Click the stamping-key, stamping reverse run an Angle, long press-key, stamping reverse slow rotation run |
| protect |  | Effective in the debugging state Click the protection key (open and close), the display icon lights up, the solenoid valve action once, then press the solenoid valve action once, the display icon is closed Long press: protection parameters of Quick interface adjustment: P1 / P2 / P3 |
| backform |  | Effective in the debugging state Click: Top mode key (open and close), light up the display icon, move the solenoid valve once, press the solenoid valve again once, and turn the display icon off Long press: top mode parameter quick interface adjustment: P4 / P5 / P6 |
| Left to send |  | Effective in the debugging state Click: Left send key (open and off), display icon light up and close, left send solenoid valve action once Long press: left send parameter shortcut interface adjustment: P 27 \ P26 \ P25 |
| Right to send |  | Effective in the debugging state Click the right key (open and close), display icon light and close, the right solenoid valve action once Long press: Right delivery parameter shortcut interface adjustment: P 30 \ P29 \ P28 |
| Grasp the buckle |  | Effective in the debugging state Click the button (open and close), display icon light and close, grasp the solenoid valve once Long press: quick interface adjustment: P10 / P11 / P12 |
| scavenging |  | Effective in the debugging state Click the blow key (on and off), display icon light and close, blow solenoid valve action once Long press: Quick interface adjustment: P19 / P20 / P21 |
| pilot lamp |  | Effective in the debugging state Click the indicator light key, display icon light, then click off, when the icon light press + or-automatically jump to P 57 parameter, can control the infrared brightness by increasing or reducing the parameter value of P 57. |
| Left material plate |  | Effective in the debugging state Click the left plate key (on and off) for the display icon Point / long press + key: add or subtract the value Point / long press-key: value Press left and right keys: vibration disc frequency / vibration disc speed switch (display) |
| The right plate |  | Effective in the debugging state Click the right plate key (on and off) for the display icon Point / long press + key: add or subtract the value Point / long press-key: value Press left and right keys: vibration disc frequency / vibration disc speed switch (display) |
| piercing |  | Effective in the debugging state Click the punch (open and close), display icon light, punch solenoid valve suction conduction; Long press: Quick adjustment interface: P 24 \ P23 \ P22 |
| revolve |  | Effective in the debugging state Press the rotation key (on and off), the display icon lights up, and the stepping motor rotates by 90 degrees Long press: Rotation parameters quick adjustment interface: P 89 |

2. Instructions for using the operation box:

2. Instructions for using the operation box:

- four. System parameter table:

- four. System parameter table:

-2-

-2-

-1-

-1-