# HL787T2 使用说明书 v1.5

1.安全上的注意事项

使用前请详细阅读本技术资料与所搭配的缝制机械说明书，配合正确使用。

1.1 (1)电源电压与工作频率：请遵照马达与控制箱铭牌所标之规格。

(2)电磁波干扰：请远离高频磁波机器或电波发射器等，以免所产生的电磁波干扰本驱动装置因而发生错误动作。 (3)接地：为防止杂讯干扰或漏电事故，请做好接地工程（包括缝纫机、马达、控制箱、定位器）。

1.2 拆卸马达或控制箱时，勿带电拔插； 控制箱里面有危险高压电，所以关闭电源后要等 1 分钟以上方可打开控制箱盖。

1.3 为保证人身安全，请在维修机械或进行穿针作业时关闭电源。

1.4  这个标示符号表示机器安装时，如有错误恐会伤害到人体或机器会受到损坏。

所以机器方面有危险性的地方会有此标志。

△这个标志符号表示有高压电等，电气方面有危险性的地方会有此标志。

2.操作盒按键说明

| 前固缝 |  | 循环：前固缝/前双固缝/前四固缝/关闭 |
| --- | --- | --- |
| 后固缝 |  | 循环：后固缝/后双固缝/后四固缝/关闭 |
| 连续固缝/调试键 |  | 短按:w 缝/自由缝循环 长按： 1. 自由缝模式下，调试模式快捷键（长按 3 秒以上） 2. 定长缝模式下，试教模式（长按 1 秒以上） |
| 剪刀/夹线功能键 |  | 短按：剪刀功能开/关 长按： 夹线功能开/关,开启后电子夹线启用 |
| 停针位/软启动功能键 |  | 短按： 上下停针位选择键 长按：软启动功能开/关 |
| 加减键 |  | 1. 速度加减键。 连续按下， 缝纫速度升高/降低， 显示屏自动切换为速度设定值。 2. 显示数段数值加减 3. 参数数值加减 4. 花样编辑数值加减 |
| 压脚模式/触发键 |  | 短按：中间停顿抬压脚/剪线后抬压脚/中间停顿剪线后抬压脚/关闭循环 长按： 1.定长缝模式下有效（开/关）（一段缝、多段缝、花样多段缝） 2.W 缝模式下自动打开 |
| 参数设定键 |  | 进入不同参数级别（参数分为 3 级） 缝纫设定界面下短按 P 键进入【参数界面】，此时显示参数列表中级别为 I 的参数。 缝纫设定界面下长按 P 键进入【密码输入界面】，输入正确维修密码后按 P 键进入【参数界面】，此时显示参数 列表中级别为 I 和 II 的参数，二级密码： 1111 |
| 确定键 |  | 1. 确认键 2. 对于带夹线功能机型，长按此按键可显示夹线力度，液晶显示“[ _3]”），再次按该键退出 |
| 左右键 |  | 缝制界面下可左右调节“3333 ” |
| 针距加减键 |  | 缝制界面下，针距大小调节 |
| 中途抬压脚高度键 |  | 缝制界面下，中途抬压脚高度调节 |
| 定长缝模式键 |  | 循环： 一段缝/多段缝/花样多段缝/自由缝 |
| 密针缝功能键 |  | 密针缝开/关 |
| 恢复出厂设置 |  | 短按：进入计件（JJ）模式 待机界面下，长按 1.5 秒 Reset 键进入恢复出厂设置界面， 按“加减键”调到“yes ”，再按“S”键 |
| 抬压脚开关键 |  | 抬压脚开/关 |
| 花样缝切换键 |  | 短按：缝制界面下可切换成花样模式（d1~d9） 长按：自由缝界面下，可进入花样编辑模式 |

3.模式设置

停针位调整模式：长按“T”键，显示“1-dj”，按“+”键，显示“2-bj”，按“S”键， 显示“P-75”，此时按“S”键会显示一个数值，前踩一下踏板，再转动手轮 到上针位位置，按“S”键保存， 上针位调整完成； 零位调整模式：长按“T”键， 显示“1-dj”，按“+”键， 显示“2-bj”，按“S”键， 显示“P-75”，在“P-75”界面按“+”键， 显示“P-6”，按“S”键， 会显示一 个数值，在此界面调整零位，若零位有轻微偏差，可以微调数值直至零位准确，调整完成后按“S”保存； 正缝针距调整模式： 长按“T”键， 显示“1-dj”，按“+”键， 显示“2-bj”，按“S”键， 显示“P-75”，再按“+”键两次会显示“P-7”，按“S”显示数值， 可调整 正逢针距，针距偏大减小数值，针距偏小加大数值； 倒缝针距调整模式： 长按“T”键， 显示“1-dj”，按“+”键， 显示“2-bj”，按“S”键， 显示“P-75”，再按“+”键三次会显示“P-8”，按“S”显示数值， 可调整 倒逢针距，针距偏大减小数值，针距偏小加大数值； 恢复出厂设置： 1：参数恢复出厂设置；2：快捷键恢复出厂设置 1、长按“P ”键进入参数界面，进入“P79”参数时会显示“0”，再按“加减键”调到“3 ”，按“S”键。 2、待机界面下，长按 1.5 秒 Reset 键进入恢复出厂设置界面，按“加减键”调到“yes ”，再按“S”键 花样编辑设置界面： 自由缝界面下长按“PATT”键 2 秒， 液晶屏显示“d1”，按+-键切换 d1-d9 花样， 选定要设置的花样号按“S”键进入设置， 液晶屏显示“1 3.0 01”，此时可以进行第一段的针数和针距值，按左右键选择要设置的项闪烁； 一段设置完成，按左右键切换到最左侧数字闪烁，按加减键切换到其他的段数设置，若 当前段数的针数为零，不能进行下一段的设置； 所有段针距和针数设置完成按“S”键保存并退回到“d1”界面， 按“TP ”键退出不保存。若要从该模式退回到正常 模式，按“P ”键退出进入正常模式。

4.故障说明

| 故障显示 | 故障内容 | 故障可能原因 | 检查项目、处理 |
| --- | --- | --- | --- |
| E011、E012 | 电机信号故障 | 电机位置传感器信号故障 | 电机插头是否接触良好；电机信号检测器件是否损坏；缝纫机手轮是否安装到位 |
| E021 E023 | 电机超负荷 | 电机堵转 电机超负荷 | 电机插头是否接触良好；机头或剪线机构是否卡死 是否缝制规格厚度以上布料；电流检测信号是否正常 |
| E022 | 剪线卡刀 | 剪线卡刀 | 检查机头或剪线机构是否卡死；检查剪线时序和位置是否正确 |
| E101 | 硬件驱动故障 | 电流检测非正常；驱动器件直通 | 系统电流检测回路是否工作正常； 驱动器件是否损坏 |
| E111、E112 | 系统电压过高 | 实际电压偏高；制动回路故障 电压检测有误 | 系统进线电压是否过高；制动电阻是否工作正常 系统电压检测回路是否工作正常 |
| E121、E122 | 系统电压过低 | 实际电压偏低；电压检测有误 | 系统进线电压是否过低；系统电压检测回路是否工作正常 |
| E131 | 电流检测回路故障 | 电流检测非正常 | 系统电流检测回路是否工作正常 |
| E133 | OZ 回路故障 | OZ 回路非正常 | 系统 OZ 回路是否工作正常 |
| E134 | DBFLT 故障 | 自动电阻回路非正常 | 自动电阻插头是否接触良好；自动电阻是否损坏 |
| E201 | 电机电流过大 | 电流检测非正常；电机运转非正常 | 系统电流检测回路是否工作正常； 电机信号是否正常 |
| E211、E212 | 电机运转非正常 | 电机运转非正常 | 电机插头是否接触良好；电机信号是否不匹配 |
| E301 | 操作盒通讯不良 | 机头操作盒通讯数据丢失 | 操作盒插头是否接触良好；操作盒器件是否损坏 |
| E302 | 操作盒 E2PROM 故障 | 操作盒内部故障 | 检查操作盒器件是否损坏 |
| E303 | SPI 通讯故障 | 操作盒内部故障 | 检查主控板器件是否损坏 |
| E304 | HMI主从芯片通信故障 | 操作盒内部故障 | 检查操作盒器件是否损坏 |
| E402 | 踏板 ID 故障 | 踏板辨识故障 | 踏板接头松动 |
| E403 | 踏板零位校正故障 | 踏板零位校正值超出范围 | 踏板损坏或者校正时踏板不是停止状态 |
| E501 | 翻抬开关故障 | 翻抬开关有效 | 放下机头或者检查翻抬开关 |
| E502 | 油量报警故障 | 油量报警故障 | 加油提示 |
| E601 | 硬件过流故障 | 针距压脚电机硬件过流 | 系统电流检测回路是否工作正常； 驱动器件是否损坏 |
| E602 | 软件过流故障 | 针距压脚电机软件过流 | 系统电流检测回路是否工作正常； 驱动器件是否损坏 |
| E603 | 电流检测回路故障 | 针距压脚电机电流检测回路 | 系统电流检测回路是否工作正常； 驱动器件是否损坏 |
| E604 | 机械找零位故障 | 针距压脚电机初始机械角度 | 针距压脚电机插头是否接触良好； |
| E605 | 电机堵转 | 针距压脚电机堵转 | 针距压脚电机插头是否接触良好； 机械是否有卡点 |
| E606 | 电机开路故障 | 针距压脚电机绕组回路 | 系统电流检测回路是否工作正常； 驱动器件是否损坏 |
| E607 | 硬件过流故障 | 剪线电机硬件过流 | 系统电流检测回路是否工作正常； 驱动器件是否损坏 |
| E608 | 软件过流故障 | 剪线电机软件过流 | 系统电流检测回路是否工作正常； 驱动器件是否损坏 |
| E609 | 电流检测回路故障 | 剪线电机电流检测回路 | 系统电流检测回路是否工作正常； 驱动器件是否损坏 |
| E610 | 机械找零位信号故障 | 剪线电机初始机械角度 | 剪线电机插头是否接触良好 |
| E611 | 电机堵转 | 剪线电机堵转 | 剪线电机插头是否接触良好；机械是否有卡点 |
| E612 | 电机开路故障 | 剪线电机绕组回路 | 系统电流检测回路是否工作正常； 驱动器件是否损坏 |
| E701 | 底线检测报警 | 底线传感器未检测到底线 | 更换底线或检查底线传感器 |
| P.oFF | 掉电显示 | 电源关闭 | 等待电源重新开通 |
| EvAL | 试用保护故障 | 试用时间到 | 联系代理商 |
| L.bob | 底线提示 | 底线计数值为负 | 更换底线后长按 S 键取消提示状态 |
| P.bob | 计件提示 | 计数值为零 | 按 S 键进入界面，长按“前固缝”键 2 秒以上取消提示状态 如需关闭计件功能，将 P35 参数数值更改为 0 |

5.系统参数表:

| 编号 | 项目 | 内容 | 设定范围 | 步距 | 默认值 | 级别 |
| --- | --- | --- | --- | --- | --- | --- |
| P-01 | 踏板最高速 | 设定踏板的最高转速 | 200~5000(rpm) | 100 | 3700 | I |
| P-02 | 软起动开关 | 开始缝纫时低速缝纫几针的功能：1~9：软起动针数 | 0~9 | 1 | 2 | I |
| P-04 | 定长缝最高速 | 设定定长缝最高速度 | 200~4000(rpm) | 100 | 3000 | I |
| P-06 | 零位针距修正值 | 针距设为 0mm 时微调该值使实际缝纫针距为 0 | 50~150 | 1 | 100 | I |
| P-07 | 正缝针距修正值 | 正缝针距放大比例（固缝针迹） | 50~150(%) | 1 | 100 | I |
| P-08 | 倒缝针距修正值 | 倒缝针距放大比例（固缝针迹） | 50~150(%) | 1 | 100 | I |
| P-09 | 倒缝开关限速速度 | 倒缝开关限速速度 | 500~1500(rpm) | 50 | 800 | I |
| P- 10 | 定长缝自编针距针数设定模式 | 0:设定各段针数即设定数值 1:定长缝设成自编时， 设定数值为花样的个数， 即总针数为花样针数×设定数值 | 0/1 | 1 | 1 | I |
| P- 12 | 抬压脚第一段角度 | 抬压脚第一段角度 | 0~100 | 1 | 29 | I |
| P- 13 | 压脚最大抬升高度（步进有效） | 剪线后压脚最大高度设置 | 0~100 | 1 | 65 | II |
| P- 14 | 压脚抬升速度（步进有效） | 压脚抬升的步进速度 | 20~500(rpm) | 10 | 300 | II |
| P- 15 | 压脚释放速度（步进有效） | 压脚释放的步进速度 | 20~500(rpm) | 10 | 300 | II |
| P- 16 | 放压脚第一段角度 | 放压脚第一段角度 | 0~100 | 1 | 31 | I |
| P- 17 | 剪线松线力度 | 剪线为电磁铁时：剪线松线力度（太小会影响剪线电磁铁吸合力度） 剪线为步进时：松线力度调节 | 0~100 | 1 | 50 | II |
| P- 18 | 前固缝衔接后固缝动作设定 | 前固缝结束立即后踩剪线不带后固缝功能设定：0：无效 1：有效 | 0/1 | 1 | 0 | I |
| P- 19 | 前固缝结束后停止 | 前固缝结束后停止设定 0：无效 1：有效 | 0/1 | 1 | 0 | I |
| P-20 | 机头按键附件功能选择 | 0：无效 1：待机时手动剪线 2：剪线后手动压脚 | 0/1/2 | 1 | 0 | I |
| P-21 | 软启动速度 1 | 软启动第 1 针速度 | 100~3000(rpm) | 50 | 400 | I |
| P-22 | 软启动速度 2 | 软启动第 2 针速度 | 100~3000(rpm) | 50 | 1000 | I |
| P-23 | 软启动速度 3 | 软启动第 3~9 针速度 | 100~3000(rpm) | 50 | 1500 | I |
| P-24 | 压脚软下降开关 | 防止损伤布料减慢压脚下降速度的设定 | 0/1 | 1 | 1 | I |
| P-25 | 抬压脚开关 | 抬压脚开关设定 0：无效 1：有效 2：剪线后有效 | 0/1/2 | 1 | 1 | I |
| P-26 | 过厚开关 | 过厚开关设定 0：无效 1：有效 | 0/1 | 1 | 0 | I |
| P-27 | 上电定位设定 | 上电时机头自动找上针位的功能设定 0：无效 1：有效 | 0/1 | 1 | 0 | I |
| P-28 | 安全开关信号模式 | 机头安全开关信号模式设定 0：常开 1：常闭 2：禁止保护 | 0/1/2 | 1 | 0 | I |
| P-29 | 压脚软下降时间 | 压脚软下降时间设定，时间越长压脚下降越慢 | 100~500（ms） | 5 | 50 | II |
| P-30 | 底线计数开关 | 0:无效 1：有效 | 0/1 | 1 | 0 | I |

| 编号 | 项目 | 内容 | 设定范围 | 步距 | 默认值 | 级别 |
| --- | --- | --- | --- | --- | --- | --- |
| P-31 | 底线初值设定 | 底线初值设定 | 200~4000（0.1m） | 20 | 1600 | I |
| P-32 | 装饰固缝中间停顿时间 | 装饰固缝中间停顿时间 | 5~500 | 5 | 50 | I |
| P-33 | 前倒缝停针位置 | 0：无效 1：有效 | 0/1 | 1 | 0 | I |
| P-34 | 标准固缝速度模式选择 | 标准固缝速度模式选择 0 自动；1 踏板控制 | 0/1 | 1 | 0 | II |
| P-35 | 计件数倍率 | 计件数倍率设定 | 0~50 | 1 | 0 | I |
| P-36 | 计件数初值设定 | 计件数初值设定 | 0~1000 | 5 | 100 | I |
| P-37 | 拨线动作时间 | 拨线动作时间 | 0~800（ms） | 10 | 40 | II |
| P-38 | 计件选择设定 | 0：计件加 1：计件减 | 0/1 | 1 | 0 | I |
| P-39 | 压脚下降关断时间 | 压脚下降关断时间 | 0~50 | 1 | 0 | II |
| P-40 | 电子松线保持时间 | 电子松线保持时间 | 0- 10 | 1 | 5 | Ⅲ |
| P-41 | 低速速度 | 踏板最低速度 | 100~500(rpm) | 10 | 200 | I |
| P-42 | 踏板加速选择 | 踏板调速功能调整 0：正常 1：加速慢 2：加速快 | 0/1/2 | 1 | 0 | I |
| P-44 | 剪线速度 | 剪线速度 | 100~500(rpm) | 10 | 300 | I |
| P-45 | 倒缝限速开关 | 倒缝开关限速处理 可以防止倒缝断针 0：无限速 1：有限速 2：一直限速 | 0/1/2 | 1 | 0 | I |
| P-46 | 放压脚延迟缝纫时间 | 为确认压脚已放下的延时 | 0~800(ms) | 10 | 100 | II |
| P-47 | 运行时膝靠压脚判断速度 | 运行时膝靠压脚判断速度 | 200~1000(rpm) | 50 | 500 | II |
| P-48 | 运行时膝靠压脚抬升高度 | 运行时膝靠压脚抬升高度 | 1~100 | 1 | 0 | II |
| P-49 | 抬压脚保持时间 | 抬压脚保持时间后强制关断 | 1~60(s) | 1 | 25 | II |
| P-50 | 抬压脚全压输出时间 | 抬压脚全压输出时间 | 0~800(ms) | 10 | 150 | II |
| P-51 | 抬压脚输出力度 | 抬压脚输出力度 | 0~100 | 1 | 40 | II |
| P-53 | 前固缝速度 | 前加固缝速度 | 100~3000(rpm) | 50 | 2200 | I |
| P-54 | 前固缝正缝补偿 | 前固缝正缝补偿系数 | 80~120 | 1 | 100 | I |
| P-55 | 前固缝倒缝补偿 | 前固缝倒缝补偿系数 | 80~120 | 1 | 100 | I |
| P-56 | 后固缝速度 | 后加固缝速度 | 100~3000(rpm) | 50 | 2200 | I |
| P-57 | 后固缝正缝补偿 | 后固缝正缝补偿系数 | 100~3000(rpm) | 50 | 2200 | I |
| P-58 | 后固缝倒缝补偿 | 后固缝倒缝补偿系数 | 100~3000(rpm) | 50 | 2200 | I |
| P-59 | 连续固缝速度 | 连续加固缝速度 | 100~3000(rpm) | 50 | 2200 | I |
| P-60 | 固缝取消限速开关 | 0：软件有限速 1：软件不限速（大针距速度高时针迹可能不稳定） | 0/1 | 1 | 0 | I |
| P-61 | 机头按钮变针距取消角度限制开关 | 0：在限制角度内变针距 1：任意角度变针距（针距可能不重合， 也可能断针） | 0/1 | 1 | 0 | I |
| P-62 | 踏板前踩开始运行的位置 | 踏板前踩开始运行的位置(相对于踏板中立时的行程) | 10~50(0.1 度) | 1 | 15 | II |
| P-63 | 踏板前踩开始加速的位置 | 踏板前踩开始加速的位置(相对于踏板中立时的行程) | 10~100(0.1 度) | 1 | 45 | II |
| P-64 | 踏板前踩开始高速的位置 | 踏板前踩开始高速的位置(相对于踏板中立时的行程) | 10~150(0.1 度) | 1 | 100 | II |
| P-65 | 踏板后踩开始抬压脚的位置 | 踏板后踩开始抬压脚的位置(相对于踏板中立时的行程） | - 100~- 10(0.1 度) | 1 | -25 | II |
| P-67 | 关闭抬压脚功能时后踩剪线的位置 | 无抬压脚功能时， 后踩剪线的位置（相对于踏板中立时的行程） | - 100~- 10(0.1 度) | 1 | -50 | II |
| P-68 | 打开抬压脚功能时后踩剪线的位置 | 有抬压脚功能时， 后踩剪线的位置（相对于踏板中立时的行程） | - 100~- 10(0.1 度) | 1 | -60 | II |
| P-69 | 下停针位 | 下停针位位置调整 | 0~240 | 1 | 175 | I |
| P-70 | 反转提针功能设定 | 0:常规模式 1:起缝变针距模式 2: 0 点起缝模式（开启反转） | 0~3 | 1 | 0 | I |
| P-71 | 反转提针角度 | 反转提针角度 | 260~360(度) | 1 | 326 | I |
| P-72 | 夹线力度调整 | 调整夹线力度大小 0：夹线功能无效 1~9：力度调节 | 0~9 | 1 | 3 | I |
| P-73 | 夹线吸合角度 | 夹线吸合角度 | 10~150(度) | 5 | 100 | I |
| P-74 | 夹线释放角度 | 夹线释放角度 | 160~300(度) | 5 | 270 | I |
| P-75 | 上停针位置 | 上停针位位置调整 | 0~240 | 1 | 33 | I |
| P-77 | 密针缝功能设置 | 0：无效 1：有效 | 0/1 | 1 | 0 | I |
| P-78 | 密针缝模式设定 | 1：起缝密针 2：结尾密针 3：首尾密针 | 1/2/3 | 1 | 2 | II |
| P-79 | 恢复出厂参数 | 特殊功能参数 5/8：恢复出厂参数 | 0~15 | 1 | 0 | I |
| P-80 | 缝纫最高速度 | 缝纫最高转速 | 300~5000(rpm) | 100 | 4000 | II |
| P-81 | 速度百分比 | 速度百分比 | 50~100 | 1 | 100 | II |
| P-82 | 试用时间 | 试用功能 0：试用功能无效； 10~1000：上电试用时间 | 0~1000(小时) | 10 | 0 | Ⅲ |
| P-83 | 主轴电机加力 | 机针穿不透布时试用；0：无效； 1~15：力度调整 | 0~15 | 1 | 0 | II |
| P-84 | 剪线加力功能 | 剪线为电磁铁时有效；0：无效； 1~15：力度调整 | 0~15 | 1 | 0 | II |
| P-85 | 剪线吸合角度 | 剪线吸合角度设定 | 150~200 | 2 | 174 | Ⅲ |
| P-86 | 剪线加力角度 | 剪线加力角度设定 | 200~300 | 2 | 260 | Ⅲ |
| P-87 | 剪线释放角度 | 剪线释放角度设定 | 300~360 | 2 | 340 | Ⅲ |
| P-88 | 松线吸合角度 | 松线吸合角度 | 0~360 | 2 | 180 | II |
| P-89 | 松线释放角度 | 松线释放角度 | 0~360 | 2 | 350 | II |
| P-90 | 语言选择设定 | 语言选择设定： 0：关闭 1：中文 2：英文 | 0~2 | 1 | 1 | II |
| P-91 | 针距锁定 | 针距是否可更改选择设定 0：允许更改 1：不允许更改 | 0~1 | 1 | 0 | II |
| P-92 | 踏板抬压脚命令时间 | 踏板后踩，抬压脚命令有效时确认时间 | 10~300(ms) | 10 | 80 | II |
| P-93 | 踏板中立位置 | 踏板中立位置微调 | - 15~15(0.1 度) | 1 | 0 | II |
| P-95 | 防鸟巢功能选择 | 防鸟巢功能选择： 0：夹线机型 1：防鸟巢机型 2：拨线机型 | 0/1/2 | 1 | 0 | II |
| P-99 | 机型选择 | 机型模式选择 | / | / | / | Ⅲ |
| P- 100 | 膝靠功能设置 | 膝靠功能设置： 0：无效 1：有效 | 0/1 | 1 | 0 | II |
| P- 101 | 膝靠传感器零位电压 | 膝靠压脚零位电压值（单位 0.01V） | 0~500 | 5 | 270 | II |
| P- 102 | 膝靠传感器最大行程电压 | 膝靠压脚最大行程电压值（单位 0.01V） | 0~500 | 5 | 60 | II |
| P- 103 | 线张力/松线功能切换 | 0 :线张力控制 1 :普通松线电磁铁控制 | 0/1 | 1 | 1 | II |
| P- 104 | 花样停针位置 | 0：无效 1：停针需走完当前花样针数 | 0/1 | 1 | 0 | II |
| P- 105 | 压脚高度传感器功能设定 | 压脚高度传感器功能设定 0：无效 1：有效 | 0/1 | 1 | 1 | II |
| P- 106 | 压脚高度传感器零位电压调整 | 压脚高度传感器零位电压调整（单位 0.01v）（压脚落下、送料牙在台板下） | 0~250 | 1 | 215 | II |
| P- 107 | 过厚检测灵敏度设定 | 过厚检测的压脚高度传感器电压设定（单位 mv）（相对于零位电压） | 0~500 | 5 | 100 | II |
| P- 108 | 剪线全压力度 | 剪线全压力度（剪线吸合时缓冲） | 50~100 | 1 | 98 | II |
| P- 109 | 压脚全压力度 | 压脚全压力度（压脚吸合时缓冲） | 50~100 | 1 | 80 | II |
| P- 110 | 布料厚度与张力控制 | 缝厚料时适当加大张力 | 0~10 | 1 | 0 | II |
| P- 111 | 过厚缝纫速度 | 过厚时限速处理 | 500~3000(rpm) | 50 | 1500 | II |
| P- 112 | 过厚针距补偿系数 | 过厚针距补偿处理 | 50~150(%) | 1 | 120 | II |
| P- 113 | 补针针距模式 | 补针针距模式 0：无效 1:有效（按 P114 设定针距） | 0/1 | 1 | 0 | I |
| P- 114 | 补针针距设定 | 针距 5mm 机型参数范围(1.0mm~5.0mm） 针距 7mm 机型参数范围(1.0mm~7.0mm） | 10～50（70） | 1 | 35 | I |
| P- 115 | 倒缝按钮功能设定 | 倒缝按钮功能设定：0：倒缝 1：密缝 2：补针 3：倒缝+补针 | 0~3 | 1 | 0 | II |
| P- 116 | 补针按钮功能设定 | 补针按钮功能设定：0：倒缝 1：密缝 2：补针 3：倒缝+补针 | 0~3 | 1 | 2 | II |
| P- 117 | 密缝针距设定 | 机头按钮密缝针距设定 | 50~150 | 1 | 110 | II |
| P- 118 | 前密针针数设定 | 前密针缝纫针数设定 | 1~10 | 1 | 1 | II |
| P- 119 | 前密针针距设定 | 前密针缝纫针距设定 | 50~150 | 1 | 110 | II |
| P- 121 | 剪线第 1 段开始角度 | 剪线第 1 段开始角度设定 | 200~300 | 2 | 230 | II |
| P- 122 | 剪线第 1 段行程 | 剪线第 1 段行程设定 | 0~100 | 1 | 55 | II |
| P- 123 | 剪线第 2 段开始角度 | 剪线第 2 段开始角度设定 | 250~360 | 2 | 330 | II |
| P- 124 | 剪线第 2 段行程 | 剪线第 2 段行程设定 | 0~100 | 1 | 75 | II |
| P- 125 | 定长缝中途停车剪线模式 | 0：无效 1：有效 | 0/1 | 1 | 0 | II |
| P- 126 | 起缝松线开关 | 0：无效 1：有效 | 0/1 | 1 | 1 | II |
| P- 127 | 起缝松线前延时 | 起缝松线前延时 | 0~1000 | 10 | 100 | II |
| P- 128 | 起缝松线动作时间 | 起缝松线动作时间 | 0~1000 | 10 | 200 | II |
| P- 129 | 显示屏背光亮度设定 | 显示屏背光亮度设定 | 0~10 | 1 | 5 | I |
| P- 135 | 中途倒缝功能选择 | 0.无效 1.有效 | 0~1 | 1 | 0 | I |
| P- 136 | 中途倒缝针数设定 | 针数设定 1~50 针 | 1~50 | 1 | 4 | I |
| P- 137 | 中途倒缝来回次数设定 | 次数设定 1~10 次 | 1~10 | 1 | 1 | I |
| P- 138 | 锁屏功能设定 | 锁屏功能设定： 0：无效 1：有效 | 0~1 | 1 | 1 | I |
| P- 139 | 锁屏时间设定 | 锁屏时间设定： 0~240 (s) | 0~240 | 1 | 2 | I |
| P- 140 | 防鸟巢勾线前延时 | 剪线结束到勾线动作的延迟时间 | 0~500ms | 5 | 50 | II |
| P- 141 | 防鸟巢勾线动作时间 | 勾线电磁铁的动作时间 | 0~500ms | 5 | 50 | II |
| P- 142 | 防鸟巢勾线后延时 | 勾线电磁铁关断的延迟时间 | 0~500ms | 5 | 50 | II |
| P- 143 | 防鸟巢勾线力度 | 调整勾线电磁铁的动作力度 | 0~100 | 1 | 100 | II |
| P- 144 | 防鸟巢吸气动作时间 | 吸气气阀的动作时间 | 0~2000ms | 10 | 250 | II |
| P- 145 | 防鸟巢挺线动作时间 | 挺线电磁铁的动作时间 | 0~500ms | 5 | 50 | II |
| P- 150 | 最大针距设定 | 最大针距设定 | 10~50（70） | 1 | 50 | II |
| P- 151 | 起缝第一针防脱线功能开关 | 0.无效 1.有效 | 0~ 1 | 1 | 0 | II |
| P- 152 | 起缝第一针防脱线针距设定 | 针距设定，参数范围 (1.0mm~5.0mm） | 10~50 | 1 | 40 | II |
| P- 153 | 机头按钮点动模式设定 | 0：无效 1：点动有效（按一次有效再按一次取消，如密缝、过厚、压脚） | 0~1 | 1 | 0 | II |
| P- 154 | 机头倒缝按钮类型选择 | 0：普通开关 1：行程开关 | 0~1 | 1 | 0 | Ⅲ |
| P- 158 | 后密针针数设定 | 后密针缝纫针数设定 | 1~10 | 1 | 1 | II |
| P- 159 | 后密针针距设定 | 后密针缝纫针距设定 | 50~150 | 1 | 110 | II |
| P- 161 | 1/2 补针按键功能设定 | 0：倒缝 1：密缝 2：1/2 补针 3：脚抬抬压脚 | 0~3 | 1 | 2 | II |
| P- 162 | 1/4 补针按键功能设定 | 0：倒缝 1：密缝 2：1/4 补针 3：脚抬抬压脚 | 0~3 | 1 | 2 | II |
| P- 166 | 前密针速度 | 前密针速度设定 | 200~3000 | 100 | 1500 | II |
| P- 167 | 后密针速度 | 后密针速度设定 | 200~3000 | 100 | 600 | II |
| P- 169 | 抬压脚位置调整 | 通过此参数可调节， 压脚放下后抬压脚曲柄的位置。 | 0~100 | 1 | 30 | II |
| P- 170 | 按速度补偿系数开关 | =0，P171~P176 参数无效； =1，P171~P176 参数生效 | 0～1 | 1 | 1 | II |
| P- 171 | 速度正缝补偿 | 速度正缝补偿系数 | 50～150 | 1 | 100 | II |
| P- 172 | 速度倒缝补偿 | 速度倒缝补偿系数 | 50～150 | 1 | 100 | II |
| P- 173 | 前固缝速度正缝补偿 | 速度正缝补偿系数 | 50～150 | 1 | 100 | II |
| P- 174 | 前固缝速度倒缝补偿 | 速度倒缝补偿系数 | 50～150 | 1 | 100 | II |
| P- 175 | 后固缝速度正缝补偿 | 速度正缝补偿系数 | 50～150 | 1 | 100 | II |
| P- 176 | 后固缝速度倒缝补偿 | 速度倒缝补偿系数 | 50～150 | 1 | 100 | II |
| P- 177 | 按键补针模式 | 0：连续补半针 1：连续补一针 2：半针 3：一针 | 0～3 | 1 | 3 | I |
| P- 178 | 按键 1/2 补针模式 | 0：连续补半针 1：连续补一针 2：半针 3：一针 | 0～3 | 1 | 3 | I |
| P- 179 | 按键 1/4 补针模式 | 0：连续补半针 1：连续补一针 2：半针 3：一针 | 0～3 | 1 | 3 | I |
| P- 180 | 按针距补偿系数开关 | =0，P181~P200 参数无效； =1，P181~P200 参数生效 | 0~1 | 1 | 1 | II |
| P- 181 | 1mm 正缝补偿 | 1mm 正缝补偿系数 | 50～150 | 1 | 100 | II |
| P- 182 | 1mm 倒缝补偿 | 1mm 倒缝补偿系数 | 50～150 | 1 | 100 | II |
| P- 183 | 2mm 正缝补偿 | 2mm 正缝补偿系数 | 50～150 | 1 | 100 | II |
| P- 184 | 2mm 倒缝补偿 | 2mm 倒缝补偿系数 | 50～150 | 1 | 100 | II |
| P- 185 | 3mm 正缝补偿 | 3mm 正缝补偿系数 | 50～150 | 1 | 100 | II |
| P- 186 | 3mm 倒缝补偿 | 3mm 倒缝补偿系数 | 50～150 | 1 | 100 | II |
| P- 187 | 4mm 正缝补偿 | 4mm 正缝补偿系数 | 50～150 | 1 | 100 | II |
| P- 188 | 4mm 倒缝补偿 | 4mm 倒缝补偿系数 | 50～150 | 1 | 100 | II |
| P- 189 | 5mm 正缝补偿 | 5mm 正缝补偿系数 | 50～150 | 1 | 100 | II |
| P- 190 | 5mm 倒缝补偿 | 5mm 倒缝补偿系数 | 50～150 | 1 | 100 | II |
| P- 191 | 6mm 正缝补偿 | 6mm 正缝补偿系数 | 50～150 | 1 | 100 | II |
| P- 192 | 6mm 倒缝补偿 | 6mm 倒缝补偿系数 | 50～150 | 1 | 100 | II |
| P- 193 | 7mm 正缝补偿 | 7mm 正缝补偿系数 | 50～150 | 1 | 100 | II |
| P- 194 | 7mm 倒缝补偿 | 7mm 倒缝补偿系数 | 50～150 | 1 | 100 | II |
| P-246 | 花样固缝功能 | 花样固缝功能设定； 0：关闭、 1：开启 | 0~1 | 1 | 0 | I |

注：表内参数为标准化，如有不符，以实际为主。

## 1. Safety instruction

Please read the operation manual and related sewing machinery datasheet carefully before correct use. 1.1 (1) Power voltage and frequence: please refer to motor and control box nameplate.

(2) Interference from electromagnetic wave:please keep far away strong magnetic or high radiation environment in order to avoid obstructions and make to misoperation.

(3) Grounding: to avoid the noise obstructions or leakage of electricity accident(inculding sewing machine,motor,control box and positioner) .

1.2 Please make sure power offat least 1min and then can open control box cover, because there are dangerous high voltage.

1.3 Please turn off the power while repairing or wearing needle in order to protect operater ’s safty.

1.4 Used where potential dangers exist.

Used where high voltage and electric danger exist.

# 2.  Operation box use

| Front Reinforcing-sewing |  | circulation：front/Reinforcing-sewing / front double/Reinforcing-sewing /front Four solid Reinforcing-sewing /turn off |
| --- | --- | --- |
| Back Reinforcing-sewing |  | circulation：back/Reinforcing-sewing / back double/Reinforcing-sewing /back Four solid Reinforcing-sewing /turn off |
| Continuous Reinforcing-sewing /debug |  | short press:circulation:w sewing Free seam long press:1. Debug mode shortcut key in free seam mode (long press for more than 3 seconds) 2. In fixed-length seam mode, try teaching mode (press for longer than 1 second) |
| Scissors function key/Thread clamp function |  | short press:Scissors function on and off. long press:Thread clamp function on and off. |
| Stop needle selection key/soft start |  | short press:Up and down needle selection key long press: starting function on and off |
| add-substract key |  | 1. Speed adjustment key. Press continuously to increase/decrease the sewing speed, and the display screen will automatically switch to the speed setting value 2.According to several numerical and subtract 3.The parameter value plus or minus 4.Pattern edit value add and subtract |
| Presser foot function cycle key/ Trigger function key Settings |  | short press:Foot presser function cycle: middle stop lifting foot/cutting line after lifting foot/middle stop cutting line after lifting foot/closing long press:1. Effective (on/off) in fixed-length seam mode (one-section seam, multi-section seam, multi-section seam self-knitting pattern) 2.W seam mode Open automatically |
| Parameters setting key |  | Enter the level of different parameters (parameters are divided into 2) sewing set interface under short press P into the parameter interface, the level for the I parameter display parameter list. Long press the "P" button in the sewing setting interface to enter the "password input interface". After entering the correct maintenance password, press the "P" button to enter the "parameters interface". At this time, the parameters of level I and II in the parameter list are displayed ,password:1111 |
| Identify key |  | 1.identify key 2. for the belt clip line function model, the strength of long press this button to display the thread clamp, LCD display "[_3]"), press the key again to exit |
| Left or right key |  | Under the sewing interface, "3333" can be adjusted left and right. |
| Needle pitch adjustment key |  | Under the sewing interface, adjust the needle pitch |
| Midway press foot height key |  | Under the sewing interface, adjust the height of the press foot midwayv |
| Fixed length sewing mode key |  | Loop: Single seam/Multi seam/Pattern Multi seam/Free seam |
| beauty stitch |  | beauty stitch on and off. |
| Reset setting |  | Short press: Enter piece rate (JJ) mode Under the standby interface, press Reset key 1.5 seconds to enter the “restoration of the factory setting interface”, press the “add-substract key” to “yes”, and then press “S” to confirm |
| Press foot lift switch |  | Open/Off the foot lift function |
| Pattern sewing switch key |  | Short press: Under the sewing interface, switch the pattern mode(d1~d9) Long press: Under the free sewing interface, enter the pattern editing mode |

Notw：The parameters in the table are standardized.If have any discrepancy,based on reality

## 3.  Mode Settings

Debug mode： 1.fter entering the normal sewing mode, press the right side 4 seconds“了”，enter 2.debug mode.If you want this pattern to return to normal mode ，press “TP ”key to exit into normal mode Stitch length Adjustment Mode：After entering sewing mode normally, long press“了”3seconds，Entering debug mode，press“+”or“-”，switch to parameter P-6， press,then the mcahine can be zero position correction,switch to parameter P-7，presscan correct the go-stitch lemgth；switch to parameter P-8，presscan correct the reverse-stitch length ，all the corrections are savec to exit by press。 Restore Factory settings： 1.Long press “ ” enter parameter interface ，enter“P79 ”will displays“0”，Press“+- ”to“3”，press “ ” 2.Under the standby interface, press Reset key 1.5 seconds to enter the “restoration of the factory setting interface ”, press the “add-substract key ”to “yes ”, and then press “S ”to confirm Pattern editing settings interface：After entering the normal sewing mode, press the “ ” Key for 2 seconds，screen display “d1”，press“+- ” toggle the d1-d9 pattern，Select the pattern to be set Press “ ” to enter the setting ，screen display “ 1 3.0 01”，In this case, the number of stitches and the stitch length of the first stage can be carried outIn this case, the number of stitches and the stitch length of the first stage can be carried out，Press left or right to select the item to be set；The first paragraph is set up，Press left or right to switch to the left，the number flashes，Press the plus or minus key to switch to other segments settings，If the current number of stitches is zero，the next setting can not be made；After all the needle distance and frame count settings are complete, press the “ ” key to save and return to the "d1" screen ，Press the“ ” key to exit without saving。If you want this pattern to return to normal mode，press “ ” key to exit into normal mode.

## 4. Error codes

| Error Code | Contents | Possible reasons | Checking and treatment |
| --- | --- | --- | --- |
| E011 E012 | Motor signal error | Motor position sensor signal failure | If electric engine plug is well contacted; if electric engine signal detecting device has been broken; if sewing machine handwheel correctly installed. |
| E021 E023 | Motor overload | motor stall motor overload | If electric engine plug is well contacted; if machine head or thread-cutting mechanism has been blocked completely;f materials are too thick; Electrical signal detection signal whether the normal. |
| E022 | Thread cutter stuck | Thread cutter stuck | Check whether the head or cutting mechanism is stuck Check whether the timing and location of the Thread trimming are correct |
| E101 | Hardware drivers fault | Current detection abnormal Driving hardware error | Current detection loop system is working properly;Whether the damage to the device driver. |
| E111 E112 | Voltage too high | High input voltage Brake circuit fault Voltage detection error | System into line voltage is too high; Braking resistance are working properly; System voltage detection circuit are working properly. |
| E121 E122 | Voltage too low | Actual low voltage Voltage detection is wrong | If the voltage on the inlet wire is too low Whether the system voltage detection circuit the normal work. |
| E131 | Current circuit fault | Current detection abnormal | Current detection loop system is working properly. |
| E133 | Oz circuit fault | Oz circuit fault | Oz circuit system is working properly. |
| E134 | DBFLT malfunction | Automatic resistance circuit failure | Brake resistor plug is in good contact Brake resistance is damaged |
| E201 | over current | Current detection error | Current detection loop system is working properly Electrical signal is normal. |
| E211 E212 | Abnormal motor operation | Current or voltage detection error | If electric engine plug is well contacted If electric engine signal |
| E301 | Communication error | Sci circuit error | if operation box plug is well contacted; if operation box components are damaged. |
| E302 | Operation inner failure | Sci circuit error | check whether the operating box is damaged. |
| E303 | SPI Communication breakdown | Sci circuit error | check whether the main control board is damaged. |
| E304 | HMI From the main chip communication failure | Sci circuit error | check whether the operating box is damaged. |
| E402 | Pedal ID fault | Pedal verification fault | Pedal connection is loosen. |
| E403 | Pedal zero position fault | The pedal zero position over range | The pedal is damaged or it is not under stop state when correction |
| E501 | Safety switch fault | Safety switch effective | Put down the head or check turned up switch. |
| E502 | Fuel fault alarm | Fuel fault alarm | Add oil Tips |
| E601 | Hardware overcurrent failure | Needle distance presser foot motor hardware overcurrent | Current detection loop system is working properly Whether the damage to the device driver. |
| E602 | Software overcurrent failure | Needle distance presser foot motor software overcurrent | Current detection loop system is working properly Whether the damage to the device driver |
| E603 | Current detection loop fault | Needle distance presser foot motor current detection circuit overcurrent | Current detection loop system is working properly Whether the damage to the device driver. |
| E604 | Mechanical zero position finding fault | Initial mechanical angle of needle distance presser foot motor | Check if the needle distance presser foot motor plug is in good conected |
| E605 | Motor stall | Needle distance presser foot motor stall | Check if the needle distance presser foot motor plug is in good contact Is there any sticking point in the machinery |
| E606 | Motor open circuit failure | Needle distance presser foot motor winding circuit | Current detection loop system is working properly Whether the damage to the device driver. |
| E607 | Hardware overcurrent failure | Thread trimming motor hardware overcurrent | Current detection loop system is working properly Whether the damage to the device driver. |
| E608 | Software overcurrent failure | Thread trimming motor software overcurrent | Current detection loop system is working properly Whether the damage to the device driver. |
| E609 | Current detection loop fault | Thread trimming motor current detection loop failure | Current detection loop system is working properly Whether the damage to the device driver. |
| E610 | Mechanical zero position finding fault | The Initial mechanical angle of thread trimming motor failure | Check if the thread trimming motor plug is in good conected |
| E611 | Motor stall | Thread trimming motor stall | Check if the thread trimming motor plug is in good contact; Is there any sticking point in the machinery |
| E612 | Motor open circuit failure | Thread trimming motor winding circuit | Current detection loop system is working properly Whether the damage to the device driver. |
| E701 | Bottom line detection alarm | The bottom line sensor does not detect the bottom line | Replace the bottom line or check the bottom line sensor |
| P.oFF | Power off Display | Power off | Wait for power supply to resume. |
| EvAL | Trial expired | Trial expired | Contact the dealer processing |
| L.bob | The bottom line tips | The bottom line count value is negative | After replacing the bottom line, press P to cancel the Alert Status |
| P.bob | Piece rate reminder | The number of counting is 0 | Press S key to enter the interface, and long press "front seam" button for more than 2 seconds to cancel the prompt state To disable the piece rate function, change the P35 parameter value to 0 |

## 5.System Parameter Table:

| Parameter code | Project | Content | Setting range | step pitch | The default value | Level |
| --- | --- | --- | --- | --- | --- | --- |
| P-01 | Sewing speed | Set sewing speed | 200~5000(rpm) | 100 | 3700 | I |
| P-02 | Soft-start switch | 1~9： Soft start stitches | 0~9 | 1 | 2 | I |
| P-04 | Fixed-length seam sewing speed | Set fixed-length seam sewing speed | 200~4000(rpm) | 100 | 3000 | I |
| P-06 | Zero pitch correction value | When the needle pitch is set to 0mm, fine the value so that the actual sewing needle pitch is zero | 50~150 | 1 | 100 | I |
| P-07 | Positive needle distance correction value | Magnification positive sewing distance (Sewing needle fixed distance) | 50~150(%) | 1 | 100 | I |
| P-08 | Anti-needle distance correction value | Enlarge reverse stitches Distance Scale (Sewing needle fixed distance) | 50~150(%) | 1 | 100 | I |
| P-09 | Back stitch speed limitation | can keep needle from breaking while backstitching | 500~1500(rpm) | 50 | 800 | I |
| P- 10 | Set the length of the stitch from the set number | 0:Set the number of stitches to set the value 1:When the length is set to self-made, the number is the number of the pattern, that is, the total number of stitches is the number of stitches × set the value | 0/1 | 1 | 1 | I |
| P- 12 | The first section angle of the presser foot lift | The first section angle of the presser foot lift | 0~100 | 1 | 29 | II |
| P- 13 | Maximum height of presser foot lifting（valid stepping） | Setting Maximum height of presser foot lifting after trimming | 0~100 | 1 | 65 | II |
| P- 14 | Speed of presser foot lifting（valid stepping） | Stepping speed of presser foot lifting | 20~500(rpm) | 10 | 400 | II |
| P- 15 | Speed of presser foot releasing（valid stepping） | Stepping speed of presser foot releasing | 20~500(rpm) | 10 | 400 | II |
| P- 16 | The first section angle of the presser foot down | The first section angle of the presser foot down | 0~100 | 1 | 31 | II |
| P- 17 | Cut and loosen duty strength | Cutting line for electromagnet: When the shear line loose line duty ratio (too small will affect the shear line electromagnet suction strength) Cutting line for step:Loose line efforts to adjust the (light) | 0~100 | 1 | 50 | II |
| P- 18 | Front reinforcing-sewing setting after the front fixed joint is connected | Immediately after the end of front reinforcing-sewing, step on the cutting line without the function setting of back reinforcing-sewing 0： unavailable 1：available | 0/1 | 1 | 0 | I |
| P- 19 | Solid after before sewing stop | 0: unavailable 1: available | 0/1 | 1 | 0 | I |
| P-20 | Function selection of machine head button | 0：invalid 1：Manual thread cutting while standby 2：Manual presser foot after trimming | 0/1/2 | 1 | 0 | I |
| P-21 | soft start speed 1 | speed of the 1 st needle of soft start | 100~3000(rpm) | 50 | 400 | I |
| P-22 | soft start speed 2 | speed of the 2 nd needle of soft start | 100~3000(rpm) | 50 | 1000 | I |
| P-23 | soft start speed 3 | speed of the 3 rd~9 th needle of soft start | 100~3000(rpm) | 50 | 1500 | I |
| P-24 | Presser foot soft lowering switch | Setting to protect damaged the fabric to slow down the presser foot lowering speed | 0/1 | 1 | 1 | I |
| P-25 | Presser foot lift function | 0: unavailable 1: available | 0/1/2 | 1 | 1 | I |
| P-26 | Switch of over thickness | Setting the function of over thickness 0：unavailable 1： available | 0/1 | 1 | 0 | I |

HL787T2 Instructions v1.4

| Parameter code | Project | Content | Setting range | step pitch | The default value | Level |
| --- | --- | --- | --- | --- | --- | --- |
| P-27 | Power on and positioning | The function setting of automatically finding the needle position when the power is on 0：unavailable 1： available | 0/1 | 1 | 0 | I |
| P-28 | signal mode for turn/lift switch | Setting of signal mode of turn/lift switch of machine head 0: always open 1: always close 2:forbid a protection | 0/1/2 | 1 | 0 | I |
| P-29 | Presser foot soft lowering time | To set presser foot soft lowering time The longer time the lower speed of the presser foot | 100~500（ms） | 5 | 50 | II |
| P-30 | Bottom line counting | 0：unavailable 1： available | 0/1 | 1 | 0 | I |
| P-31 | Bottom line initial value setting | Bottom line initial value setting | 200~4000（0.1m） | 20 | 1600 | I |
| P-32 | Intermediate stopping time for decorative fastening seams | Intermediate stopping time for decorative fastening seams | 5~500 | 5 | 50 | I |
| P-33 | Front backstitch stop position | 0：unavailable 1： available | 0/1 | 1 | 0 | I |
| P-34 | Standard joint speed mode selection | Select 0 automatic for standard joint speed mode; 1 pedal control | 0/1 | 1 | 0 | II |
| P-35 | multiple ratio of counting by piece | Setting of multiple ratio of counting by piece | 0~50 | 1 | 0 | I |
| P-36 | Setting initial value of counting by piece | Setting initial value of counting by piece | 0~1000 | 5 | 100 | I |
| P-37 | Time of thread wiper | Time of thread wiper | 0~800（ms） | 10 | 40 | II |
| P-38 | Setting function of choosing counting by piece | 0：add 1：subtract | 0/1 | 1 | 0 | I |
| P-39 | Turn off time before presser foot soft drop | Turn off time before presser foot soft drop | 0~50 | 1 | 12 | I |
| P-40 | Electronic loose wire holding time | Electronic loose wire holding time | 0- 10 | 1 | 5 | Ⅲ |
| P-41 | Low speed | The lowest speed of pedal | 100~500(rpm) | 10 | 200 | I |
| P-42 | Pedal Acceleration Options | Pedal speed adjustment 0: normal 1: Slow acceleration 2: Quick acceleration | 0/1/2 | 1 | 0 | I |
| P-44 | thread-cutting speed | thread-cutting speed | 100~500(rpm) | 10 | 100 | I |
| P-45 | Back-tracking speed limit function | Back-tracking speed processing can prevent reverse sewing needle breakage 0:1: infinite speed limit 2: the speed limit | 0/1/2 | 1 | 0 | I |
| P-46 | presser foot lifting delays sewing | delay with presser foot lowered | 0~800(ms) | 10 | 100 | II |
| P-47 | When running, the knee presses the foot to judge the speed | When running, the knee presses the foot to judge the speed | 200~1000(rpm) | 50 | 500 | II |
| P-48 | When running the knee is raised by the foot | When running the knee is raised by the foot | 1~100 | 1 | 0 | II |
| P-49 | Press foot holding time | Lift the pressure foot after holding time to force off | 1~60(s) | 1 | 8 | II |
| P-50 | output time of total pressure of presser foot lifting | output time of total pressure of presser foot lifting | 0~800(ms) | 10 | 150 | II |
| P-51 | output duty strength of presser foot lifting hold time of presser foot lifting | output duty strength of presser foot lifting hold time of presser foot lifting | 0~100 | 1 | 40 | II |
| P-53 | starting reinforcing-sewing speed | starting reinforcing-sewing speed | 100~3000(rpm) | 50 | 2000 | I |
| P-54 | Compensation coefficient of front tacking sewing | Compensation coefficient of front tacking sewing | 80~120 | 1 | 100 | I |
| P-55 | Compensation coefficient of front back tacking sewing | Compensation coefficient of front back tacking sewing | 80~120 | 1 | 100 | I |
| P-56 | ending reinforcing-sewing speed | ending reinforcing-sewing speed | 100~3000(rpm) | 50 | 2000 | I |
| P-57 | starting reinforcing-sewing of front tacking sewing | starting reinforcing-sewing of front tacking sewing | 80~120 | 1 | 100 | I |
| P-58 | starting reinforcing-sewing of front back tacking sewing | starting reinforcing-sewing of front back tacking sewing | 80~120 | 1 | 100 | I |
| P-59 | Continuous reinforcing-sewing speed | Continuous reinforcing-sewing speed | 100~3000(rpm) | 50 | 2000 | I |
| P-60 | Bartack stitch cancel speed limit | 0：The software has speed limited 1：No speedlimited | 0/1 | 1 | 0 | I |
| P-61 | A switch to cancel the angle limit when change stitch length | 0：Change the stitch length within the limiting angle 1：Vhange stitch length at any angle （The stitch length may not coincide,or the needle may break） | 0/1 | 1 | 0 | I |
| P-62 | Pedal travel upon start | Pedal position upon start (Travel relative to medium pedal) | 10~50(0.1°) | 1 | 15 | II |
| P-63 | Pedal travel upon acceleration | Pedal position upon start acceleration (Travel relative to medium pedal) | 10~100(0.1°) | 1 | 45 | II |
| P-64 | Pedal travel at highest rotation speed | Pedal position at highest rotating speed (Travel relative to medium pedal) | 10~150(0.1°) | 1 | 100 | II |
| P-65 | Pedal travel upon presser foot lift | Pedal position upon pedal lift (Travel relative to medium pedal) | - 100~- 10(0.1°) | 1 | -25 | II |
| P-67 | Pedal travel 1 upon thread trimming | Pedal position upon start trimming without presser foot function Travel relative to medium pedal | - 100~- 10(0.1°) | 1 | -50 | II |
| P-68 | Pedal travel 2 upon thread trimming | Pedal position upon start thread trimming with presser foot function (Travel relative to medium pedal) | - 100~- 10(0.1°) | 1 | -60 | II |
| P-69 | Down needle positioning position | To adjust down needle position | 0~240 | 1 | 175 | I |
| P-70 | Reverse needle lift function setting | Selection of sewing needle pitch mode: 0: Normal mode 1: Starting sewing with varying needle pitch mode 2: Starting sewing at 0 point mode (enabling reverse) | 0~2 | 1 | 0 | I |
| P-71 | Reversal of needle lift angle | Reversal of needle lift angle | 260~360° | 1 | 340 | I |
| P-72 | Thread clamp strength adjustment | Adjust the thread clamp strength size 0: Cli p line function is invalid 1~9: Three Intensity Adjustment | 0~9 | 1 | 3 | I |
| P-73 | Thread pressing actuation angle | Thread pressing actuation angle | 10~150° | 5 | 100 | I |
| P-74 | Thread pressing release angle | Thread pressing release angle | 160~300° | 5 | 270 | I |
| P-75 | Upper stop position | Upper stop pin position adjustment | 0~240 | 1 | 33 | I |
| P-77 | Setting of beauty sewing | 0：unavailable 1： available | 0/1 | 1 | 0 | I |
| P-78 | Beauty sewing（close sewing）mode setting | 1：Close stitch on start sewing 2：Close stitch on end-sewing 3：Close stitch on both start nad end sewing | 1/2/3 | 1 | 2 | II |
| P-79 | return to factory-set parameter | Special functional parameters 5/8：Restore factory parameters | 0~15 | 1 | 0 | I |
| P-80 | Maximum sewing speed | The highest sewing speed | 300~5000(rpm) | 100 | 4000 | II |
| P-81 | Percentage of speed | Percentage of speed | 50~100 | 1 | 100 | II |
| P-82 | Trial period | Trial function 0: Invalid trial function 10- 1000: Power on trial time | 0~1000(hour) | 10 | 0 | Ⅲ |
| P-83 | Spindle motor loading | Try if the needle can't penetrate the cloth. 0: invalid; 1~15: intensity adjustment | 0~15 | 1 | 0 | II |
| P-84 | Function of cutting line and adding force | Effective when the cutting wire is an electromagnet; 0: invalid; 1~15: intensity adjustment | 0~15 | 1 | 0 | II |
| P-85 | Thread cutting and suction angle | Setting the angle of thread cutting and suction | 150~200 | 2 | 174 | Ⅲ |
| P-86 | Cutting and applying force angle | Setting the angle of thread cutting and force application | 200~300 | 2 | 260 | Ⅲ |
| P-87 | Thread cutting release angle | Setting the angle of thread trimming release | 300~360 | 2 | 340 | Ⅲ |
| P-88 | Loose line suction Angle | Loose line suction Angle | 0~360 | 2 | 180 | II |
| P-89 | Loose line release Angle | Loose line release Angle | 0~360 | 2 | 350 | II |
| P-90 | Language selection setting | Setting of language：0：turn off 1：Chinese 2：English | 0~2 | 1 | 1 | II |
| P-91 | Gauge Lock | Change pitch selection 0:Changes Allowed 1:Allowed to change | 0~1 | 1 | 0 | II |
| P-92 | Pedal presser foot lift confirm time | Pedal presser foot lift confirm time | 10~300(ms) | 10 | 80 | II |
| P-93 | The neutral position of the pedal | Trimming the neutral position of the pedal | - 15~15(0.1degree) | 1 | 0 | II |
| P-95 | Anti-nest function selection | Anti-nest function selection：0：hread clamp type 1：Anti-next type 2：thread wiper type | 0/1/2 | 1 | 0 | II |
| P-99 | Machine mode selection | Machine mode selection | / | / | / | Ⅲ |
| P- 100 | Knee by function setting | Knee by function setting：0：unavailable 1：available | 0/1 | 1 | 1 | II |
| P- 101 | The voltage at the zero position of the knee sensor | The voltage at zero position of knee by’s presser foot（Unit:0.01V） | 0~500 | 5 | 270 | II |
| P- 102 | Knee sensor’s voltage of maximun travel distance | Knee sendor’s voltage of maximun stravel distance（unit:0.01V） | 0~500 | 5 | 60 | II |
| P- 103 | Line tension / loose line function switch | 0 :Line tension control 1 :Ordinary loose line electromagnet control | 0/1 | 1 | 1 | II |
| P- 104 | Pattern stop position | 0：unavailable 1：Stop the needle need to go through the current number of pattern stitches | 0/1 | 1 | 0 | II |
| P- 105 | Presser foot height sensor function setting | Presser foot height sensor function setting 0：unavailable 1： available | 0/1 | 1 | 1 | II |
| P- 106 | The voltage adjustment at zero position of presser foot height sensor | The voltage adjustment at zero position of presser foot height sensor（unit:0.01v） （presser foot drop,feed dog is under the table） | 0~250 | 1 | 215 | II |
| P- 107 | Overthickness detection sensitivity setting | Presser foot height sensor’s coltage setting of overthickness detection Unit:mv）（Relative to the zero position voltage） | 0~500 | 5 | 100 | II |
| P- 108 | Full pressure duty strength of thread trimming | Full pressure duty strength of thread trimming(Buffering during the thread trimming and closing) | 50~100 | 1 | 80 | II |
| P- 109 | Force adjustment of electromagnet for presser foot lifiting | Force adjustment of electromagnet for presser foot lifiting（Cushion when press foot is absorbed） | 50~100 | 1 | 80 | II |
| P- 110 | Fabric thickness and tension control | Increase tension appropriately when sewing thick materials | 0~10 | 1 | 0 | II |
| P- 111 | Overthickness sewing speed | Limit speed of overthickness | 500~3000(rpm) | 50 | 1500 | II |
| P- 112 | Overthickness stitch offset coefficient | Overthickness stitch offset deals | 50~150(%) | 1 | 120 | II |
| P- 113 | Needle compensate stitch length mode | Needle compensate stitch length mode0：Invaild:vaild（press P114 to set stitch length） | 0/1 | 1 | 0 | I |
| P- 114 | Needle compensate stitch length setting | Parameter range of Stitch length 5mm type(1.0mm~5.0mm） Parameter range of stitch length 5mm type(1.0mm~7.0mm） | 10～50（70） | 1 | 35 | I |
| P- 115 | Reverse sewing button function setting | 0: reverse sewing 1: tightly sewn 2: reverse sewing 3:reverse sewing+reverse sewing | 0~3 | 1 | 0 | II |
| P- 116 | fill needle button function setting | 0: reverse sewing 1: tightly sewn 2: reverse sewing 3:reverse sewing+reverse sewing | 0~3 | 1 | 2 | II |
| P- 117 | Dense needle pitch setting | Head button stitch stitch set | 50~150 | 1 | 110 | II |
| P- 118 | Number of pretty sewing needle setting | Number of pretty sewing needle setting | 1~10 | 1 | 1 | II |
| P- 119 | Pretty sewing needle distance setting | Pretty sewing needle distance setting | 50~150 | 1 | 110 | II |
| P- 121 | Start trimming angle of Paragraph 1 | Start trimming angle of Paragraph 1 | 200~300 | 2 | 230 | II |
| P- 122 | Paragraph 1 stroke of trimming | Paragraph 1 stroke of trimming | 0~100 | 1 | 40 | II |
| P- 123 | Start trimming angle of Paragraph 2 | Start trimming angle of Paragraph 2 | 250~360 | 2 | 330 | II |
| P- 124 | Paragraph 2 stroke of trimming | Paragraph 2 stroke of trimming | 0~100 | 1 | 65 | II |
| P- 125 | Stop and trimming during fixed-length sewing | 0：invaild 1：vaild | 0/1 | 1 | 0 | II |
| P- 126 | loose thread enable when start sewing | 0：invaild 1：vaild | 0/1 | 1 | 1 | II |
| P- 127 | Delay before start-sewing loose thread | Delay before start-sewing loose thread | 0~1000 | 10 | 100 | II |
| P- 128 | Action time of loose thread when start-sewing | Action time of loose thread when start-sewin | 0~1000 | 10 | 200 | II |
| P- 129 | Display backlight brightness Setting | Display backlight brightness Setting | 0~10 | 1 | 5 | I |
| P- 135 | Midway reverse function selection | 0: unavailable 1: available | 0~1 | 1 | 0 | I |
| P- 136 | The number of midway stitch set | Pin setting | 1~50 | 1 | 4 | I |
| P- 137 | Seam midway down the number of times back and forth set | Set times | 1~10 | 1 | 1 | I |
| P- 138 | Lock screen function setting | Lock screen function setting：0：invalid 1：valid | 0~1 | 1 | 1 | I |
| P- 139 | Lock screen time setting | Lock screen time setting： 0~240 (s) | 0~240 | 1 | 2 | I |
| P- 140 | Delay time before thread hooking of Anti-nest | The delay time between the end of trimming and the hook | 0~500ms | 5 | 50 | II |
| P- 141 | Thread hooking time of Anti-nest | The action time of the hooked electromagnet | 0~500ms | 5 | 50 | II |
| P- 142 | Delay after Anti-nest hook | The delay time of the hook electromagnet turning off | 0~500ms | 5 | 50 | II |
| P- 143 | Thread hooking duty ratio of Anti-nest | Adjust the action force of the hooked electromagnet | 0~100 | 1 | 100 | II |
| P- 144 | Anti-nest suction work time | The work time of the suction valve | 0~2000ms | 10 | 250 | II |
| P- 145 | Thread-straight work time of Anti-nest | The work time of a thread-straight solenoid | 0~500ms | 5 | 50 | II |
| P- 150 | Maximum needle pitch setting | Maximum needle pitch setting | 10~50（70） | 1 | 50 | II |
| P- 151 | First off-line prevented stitch length | 0.invaild 1.vaild | 0~1 | 1 | 0 | II |
| P- 152 | First off-line prevented stitch length setting | Stitch length setting Parameter range (1.0mm~5.0mm） | 10~50 | 1 | 40 | II |
| P- 153 | Headbutton inching mode setting | 0：Invalid 1：valid（Press once valid ,press again cancel, such as close sewing, over thickness, presser foot） | 0~1 | 1 | 0 | II |
| P- 154 | Machine head reverse sewing button type selection | 0: Normal switch 1: Travel switch | 0~1 | 1 | 0 | Ⅲ |
| P- 158 | Setting the number of the rear dense needle | Setting the number of the rear dense needle | 1~10 | 1 | 1 | II |
| P- 159 | Setting the needle spacing of the rear dense needle | Setting the needle pitch for back stitch sewing | 50~150 | 1 | 110 | II |
| P- 161 | 1/2 Needle filling button function setting | 0: Reverse sewing 1: Tight sewing 2:1/2 Needle filling 3: Foot lifting and pressing | 0~3 | 1 | 2 | II |
| P- 162 | 1/4 needle filling button function setting | 0: Reverse sewing 1: Tight sewing 2:1/2 Needle filling 3: Foot lifting and pressing | 0~3 | 1 | 2 | II |
| P- 166 | Front dense needle speed | Front dense needle speed setting | 200~3000 | 100 | 1500 | II |
| P- 167 | Rear dense needle speed | Rear dense needle speed setting | 200~3000 | 100 | 600 | II |
| P- 169 | Presser foot position adjustment | This parameter can be used to adjust the position of the presser foot crank when the presser foot is lowered | 0~100 | 1 | 30 | II |
| P- 170 | Compensate system enablement by speed | =0 ，P171~P176 Invalid parameter=1，P171~P176 Invalid parameter | 0～1 | 1 | 1 | II |
| P- 171 | Speed clockwise sewing compensation | Speed clockwise sewing compensate system | 50～150 | 1 | 100 | II |
| P- 172 | Speed anti-clockwise sewing compensation | Speed anti-clockwise sewing compensate system | 50～150 | 1 | 100 | II |
| P- 173 | Starting reinforcing-sewing speed Clockwise sewing compensation | Speed Clockwise sewing compensation coefficient | 50～150 | 1 | 100 | II |
| P- 174 | Starting reinforcing-sewing speed Anti-clockwise sewing compensation | Speed Anti-clockwise sewing compensation coefficient | 50～150 | 1 | 100 | II |
| P- 175 | ending reinforcing-sewing speed Clockwise sewing compensation | Speed Clockwise sewing compensation coefficient | 50～150 | 1 | 100 | II |
| P- 176 | ending reinforcing-sewing speed Anti-clockwise sewing compensation | Speed Anti-clockwise sewing compensation coefficient | 50～150 | 1 | 100 | II |
| P- 177 | Button needle filling mode | 0: Continuous half stitch 1: Continuous one stitch 2: Half stitch 3: One stitch | 0～3 | 1 | 3 | I |
| P- 178 | Button 1/2 needle filling mode | 0: Continuous half stitch 1: Continuous one stitch 2: Half stitch 3: One stitch | 0～3 | 1 | 3 | I |
| P- 179 | Button 1/4 needle filling mode | 0: Continuous half stitch 1: Continuous one stitch 2: Half stitch 3: One stitch | 0～3 | 1 | 3 | I |
| P- 180 | Compensate system enablement by stitch length | =0 ，P181~P200 Invalid parameter =1 ，P181~P200 Invalid parameter | 0~1 | 1 | 1 | II |
| P- 181 | 1mm Clockwise sewing compensation | 1mm Clockwise sewing compensation coefficient | 50～150 | 1 | 100 | II |
| P- 182 | 1mm Anti-clockwise sewing compensation | 1mm Anti-clockwise sewing compensation coefficient | 50～150 | 1 | 100 | II |
| P- 183 | 2mm Clockwise sewing compensation | 2mm Clockwise sewing compensation coefficient | 50～150 | 1 | 100 | II |
| P- 184 | 2mm Anti-clockwise sewing compensation | 2mm Anti-clockwise sewing compensation coefficient | 50～150 | 1 | 100 | II |
| P- 185 | 3mm Clockwise sewing compensation | 3mm Clockwise sewing compensation coefficient | 50～150 | 1 | 100 | II |
| P- 186 | 3mm Anti-clockwise sewing compensation | 3mm Anti-clockwise sewing compensation coefficient | 50～150 | 1 | 100 | II |
| P- 187 | 4mm Clockwise sewing compensation | 4mm Clockwise sewing compensation coefficient | 50～150 | 1 | 100 | II |
| P- 188 | 4mm Anti-clockwise sewing compensation | 4mm Anti-clockwise sewing compensation coefficient | 50～150 | 1 | 100 | II |
| P- 189 | 5mm Clockwise sewing compensation | 5mm Clockwise sewing compensation coefficient | 50～150 | 1 | 100 | II |
| P- 190 | 5mm Anti-clockwise sewing compensation | 5mm Anti-clockwise sewing compensation coefficient | 50～150 | 1 | 100 | II |
| P- 191 | 6mm Clockwise sewing compensation | 6mm Clockwise sewing compensation coefficient | 50～150 | 1 | 100 | II |
| P- 192 | 6mm Anti-clockwise sewing compensation | 6mm Anti-clockwise sewing compensation coefficient | 50～150 | 1 | 100 | II |
| P- 193 | 7mm Clockwise sewing compensation | 7mm Clockwise sewing compensation coefficient | 50～150 | 1 | 100 | II |
| P- 194 | 7mm Anti-clockwise sewing compensation | 7mm Anti-clockwise sewing compensation coefficient | 50～150 | 1 | 100 | II |
| P-246 | Pattern fixed sewing function | Pattern fixed sewing function; 0: Off, 1: On | 0~1 | 1 | 0 | I |

Notw：The parameters in the table are standardized.If have any discrepancy,based on reality