| front Reinforcing-sewing |  | circulation：front Reinforcing-sewing / front double Reinforcing-sewing /front Four solid Reinforcing-sewing /turn off |
| --- | --- | --- |
| back Reinforcing-sewing |  | circulation：back Reinforcing-sewing / back double Reinforcing-sewing /back Four solid Reinforcing-sewing /turn off |
| Fixed seam pattern key |  | After this function is enabled, the front and rear fixed seams can be sewn according to the self-editing pattern |
| Continuous Reinforcing-sewing |  | circulation:w sewing Free seam |
| Scissors function key |  | Scissors function on and off |
| Stop needle selection key |  | Up and down needle selection key |
| The speed and key |  | Speed up. Keeping pressing to increase speed, the display will automatically switch to speed set. Simple seam, free seam effective. |
| The speed reduction key |  | Speed down. Keeping pressing to lower speed, the display will automatically switch to speed set. Simple seam, free seam effective. |
| Presser foot function cycle key |  | Foot presser function cycle: middle stop lifting foot/cutting line after lifting foot/middle stop cutting line after lifting foot/closing |
| Parameters setting key |  | 1. Enter the level of different parameters (parameters are divided into 3) sewing set interface under short press P into the parameter interface, the level for the I parameter display parameter list. Long press the "P" button in the sewing setting interface to enter the "password input interface". After entering the correct maintenance password, press the "P" button to enter the "parameters interface". At this time, the parameters of level I and II in the parameter list are displayed |
| Identify key |  | - identify key - for the belt clip line function model, the strength of long press this button to display the thread clamp, LCD display "[_7]"), press the key again to exit |
| Left or right key |  | 1. Under normal sewing interface display around several switch 2. Figure shows around several switch gap setting |
| add-substract key |  | Under normal sewing interface 1. Adjust the stitch size 2. According to several numerical and subtract (S key seam free mode into the display number segment) 3. The parameter value plus or minus 4. Pattern edit value add and subtract |
| Section stitch pattern self-knitting mode |  | Circulation: multi - section self - stitch/free stitch Able to set several sections of number sewing, each section can set the range of 0 ~ 99 stitches, each section of stitch length pattern can be knitting |
| Trigger function key Settings |  | 1. Effective (on/off) in fixed-length seam mode (one-section seam, multi-section seam, multi-section seam self-knitting pattern) 2. W seam mode Open automatically |
| Three - segment mode switching |  | Short press to switch one-stitch sewing/multi-stitch sewing/free sewing |
| Motor Angle adjustment/trial teaching function |  | 1. Debug mode shortcut key in free seam mode (long press for more than 3 seconds) 2. In fixed-length seam mode, try teaching mode (press for longer than 1 second) |

3. Mode Settings

Debug mode：After entering the normal sewing mode, press the right side 4 seconds“”，enter debug mode.If you want this pattern to return to normal mode，press “”key to exit into normal mode Stitch length Adjustment Mode：After entering sewing mode normally, long press“”3seconds，Entering debug mode，press“”or“”，switch to parameter P-6，press,then the mcahine can be zero position correction,switch to parameter P-7，presscan correct the go-stitch lemgth；switch to parameter P-8，presscan correct the reverse-stitch length，all the corrections are savec to exit by press。 Restore Factory settings：Long press “” enter parameter interface，enter“P79”will displays“0”，Press“+-”to“8”，press“” display “no”，Then press “+-”to“yes”，press “” Pattern editing settings interface：After entering the normal sewing mode, press the “ ” Key for 2 seconds，screen display “d1”，press“+-” toggle the d1-d9 pattern，Select the pattern to be set Press “” to enter the setting，screen display “1 3.0 01”，In this case, the number of stitches and the stitch length of the first stage can be carried outIn this case, the number of stitches and the stitch length of the first stage can be carried out，Press left or right to select the item to be set；The first paragraph is set up，Press left or right to switch to the left，the number flashes，Press the plus or minus key to switch to other segments settings，If the current number of stitches is zero，the next setting can not be made；After all the needle distance and frame count settings are complete, press the “” key to save and return to the "d1" screen，Press the“” key to exit without saving。If you want this pattern to return to normal mode，press “” key to exit into normal mode.

2. Operation box use

- 5. System Parameter Table:

|  | Project | Content | Setting range | step pitch | The default value | Level |
| --- | --- | --- | --- | --- | --- | --- |
| P-01 | Sewing speed | Set sewing speed | 200~5000(rpm) | 100 | 3700 | I |
| P-02 | Soft-start function | 1~9： Soft start stitches | 0~9 | 1 | 2 | I |
| P-04 | Fixed-length seam sewing speed | Set fixed-length seam sewing speed | 200~4000(rpm) | 100 | 3000 | I |
| P-06 | Zero pitch correction value | When the needle pitch is set to 0mm, fine the value so that the actual sewing needle pitch is zero | 50~150 | 1 | 100 | I |
| P-07 | Positive needle distance correction value | Magnification positive sewing distance (Sewing needle fixed distance) | 50~150(%) | 1 | 100 | I |
| P-08 | Anti-needle distance correction value | Enlarge reverse stitches Distance Scale (Sewing needle fixed distance) | 50~150(%) | 1 | 100 | I |
| P-09 | Back stitch speed limitation | can keep needle from breaking while backstitching | 500~1500(rpm) | 50 | 800 | I |
| P-10 | Set the length of the stitch from the set number | 0:Set the number of stitches to set the value 1:When the length is set to self-made, the number is the number of the pattern, that is, the total number of stitches is the number of stitches × set the value | 0/1 | 1 | 1 | I |
| P-12 | Height of middle presser foot lifting（valid stepping） | Setting height of presser foot lifting when sewing half | 0~100 | 1 | 60 | II |
| P-13 | Maximum height of presser foot lifting（valid stepping） | Setting Maximum height of presser foot lifting after trimming | 0~100 | 1 | 65 | II |
| P-14 | Speed of presser foot lifting（valid stepping） | Stepping speed of presser foot lifting | 20~300(rpm) | 10 | 200 | II |
| P-15 | Speed of presser foot releasing（valid stepping） | Stepping speed of presser foot releasing | 20~300(rpm) | 10 | 120 | II |
| P-16 | Loose wire electromagnet duty cycle | Loose wire electromagnet intensity adjustment(Thin cloth) | 0~100 | 1 | 8 | II |
| P-17 | Cut and loosen duty cycle | Cutting line for electromagnet: When the shear line loose line duty ratio (too small will affect the shear line electromagnet suction strength) Cutting line for step: Loose line efforts to adjust the (light) | 0~100 | 1 | 50 | II |
| P-18 | Front reinforcing-sewing setting after the front fixed joint is connected | Immediately after the end of front reinforcing-sewing, step on the cutting line without the function setting of back reinforcing-sewing 0：unavailable 1：available | 0/1 | 1 | 0 | I |
| P-19 | Solid after before sewing stop | 0: unavailable 1: available | 0/1 | 1 | 0 | I |
| P-20 | Function selection of machine head button | 0：invalid 1：Manual thread cutting while standby 2：Manual presser foot after trimming | 0/1/2 | 1 | 0 | I |
| P-21 | soft start speed 1 | speed of the 1 st needle of soft start | 100~3000(rpm) | 50 | 400 | I |
| P-22 | soft start speed 2 | speed of the 2 nd needle of soft start | 100~3000(rpm) | 50 | 1000 | I |
| P-23 | soft start speed 3 | speed of the 3 rd~9 th needle of soft start | 100~3000(rpm) | 50 | 1500 | I |
| P-24 | Presser foot soft lowering function | Setting to protect damaged the fabric to slow down the presser foot lowering speed | 0/1 | 1 | 1 | I |
| P-25 | Presser foot lift function | 0: unavailable 1: available | 0/1/2 | 1 | 1 | I |
| P-26 | Function of over thickness | Setting the function of over thickness 0：unavailable 1： available | 0/1 | 1 | 0 | I |
| P-27 | Power on and positioning | The function setting of automatically finding the needle position when the power is on 0：unavailable 1： available | 0/1 | 1 | 0 | I |
| P-28 | signal mode for turn/lift switch | Setting of signal mode of turn/lift switch of machine head 0: always open 1: always close 2:forbid a protection | 0/1/2 | 1 | 0 | I |
| P-29 | Presser foot soft lowering time | To set presser foot soft lowering time The longer time the lower speed of the presser foot | 100~500（ms） | 5 | 50 | II |
| P-30 | Baseline count enable | 0：unavailable 1： available | 0/1 | 1 | 0 | I |
| P-31 | Baseline initial value setting | Baseline initial value setting | 20~4000（0.1m） | 20 | 1600 | I |
| P-34 | Standard joint speed mode selection | Select 0 automatic for standard joint speed mode; 1 pedal control | 0/1 | 1 | 0 | II |
| P-35 | multiple ratio of counting by piece | Setting of multiple ratio of counting by piece | 0~50 | 1 | 0 | I |
| P-36 | Setting initial value of counting by piece | Setting initial value of counting by piece | 0~1000 | 5 | 100 | I |
| P-37 | Time of thread wiper | Time of thread wiper | 0~800（ms） | 10 | 40 | II |
| P-38 | Setting function of choosing counting by piece | 0：add 1：subtract | 0/1 | 1 | 0 | I |
| P-41 | Low speed | The lowest speed of pedal | 100~500(rpm) | 10 | 200 | I |
| P-42 | Pedal curve selection | Pedal speed adjustment 0: normal 1: Slow acceleration 2: Quick acceleration | 0/1/2 | 1 | 2 | I |
| P-44 | thread-cutting speed | thread-cutting speed | 100~500(rpm) | 10 | 300 | I |
| P-45 | Back-tracking speed limit function | Back-tracking speed processing can prevent reverse sewing needle breakage 0:1: infinite speed limit 2: the speed limit | 0/1/2 | 1 | 0 | I |
| P-46 | presser foot lifting delays sewing | delay with presser foot lowered | 0~800(ms) | 10 | 100 | II |
| P-47 | When running, the knee presses the foot to judge the speed | When running, the knee presses the foot to judge the speed | 200~1000(rpm) | 50 | 500 | II |
| P-48 | When running the knee is raised by the foot | When running the knee is raised by the foot | 1~100 | 1 | 0 | II |
| P-49 | Press foot holding time | Lift the pressure foot after holding time to force off | 1~60(s) | 1 | 25 | II |
| P-50 | output time of total pressure of presser foot lifting | output time of total pressure of presser foot lifting | 0~800(ms) | 10 | 150 | II |
| P-51 | output duty cycle of presser foot lifting hold time of presser foot lifting | output duty cycle of presser foot lifting hold time of presser foot lifting | 0~100 | 1 | 40 | II |
| P-53 | starting reinforcing-sewing speed | starting reinforcing-sewing speed | 100~3000(rpm) | 50 | 2200 | I |
| P-56 | ending reinforcing-sewing speed | ending reinforcing-sewing speed | 100~3000(rpm) | 50 | 2200 | I |
| P-59 | Continuous reinforcing-sewing speed | Continuous reinforcing-sewing speed | 100~3000(rpm) | 50 | 2200 | I |
| P-60 | Bartack stitch cancel speed limit | 0：The software has speed limited 1：No speedlimited （The stitch length may be unstable when stitch length is large and the speed is high） | 0/1 | 1 | 0 | I |
| P-61 | A switch to cancel the angle limit when change stitch length | 0：Change the stitch length within the limiting angle 1：Vhange stitch length at any angle （The stitch length may not coincide,or the needle may break） | 0/1 | 1 | 0 | I |
| P-62 | Pedal travel upon start | Pedal position upon start Travel relative to medium pedal | 10~50(0.1°) | 1 | 25 | II |
| P-63 | Pedal travel upon acceleration | Pedal position upon start acceleration Travel relative to medium pedal | 10~100(0.1°) | 1 | 50 | II |
| P-64 | Pedal travel at highest rotation speed | Pedal position at highest rotating speed Travel relative to medium pedal | 10~150(0.1°) | 1 | 110 | II |
| P-65 | Pedal travel upon presser foot lift | Pedal position upon pedal lift Travel relative to medium pedal | -100~-10(0.1°) | 1 | -30 | II |
| P-67 | Pedal travel 1 upon thread trimming | Pedal position upon start trimming without presser foot function Travel relative to medium pedal | -100~-10(0.1°) | 1 | -30 | II |
| P-68 | Pedal travel 2 upon thread trimming | Pedal position upon start thread trimming with presser foot function Travel relative to medium pedal | -100~-10(0.1°) | 1 | -60 | II |
| P-69 | Down needle positioning position | To adjust down needle position | 0~240 | 1 | 175 | I |
| P-70 | Reverse needle lift function | Reversal of needle lift function after thread trimming 0: unavailable 1: available | 0/1 | 1 | 0 | I |
| P-71 | Reversal of needle lift angle | Reversal of needle lift angle | 0~45° | 1 | 20 | I |
| P-72 | Thread clamp strength adjustment | Adjust the thread clamp strength size 0: Cli p line function is invalid 1~9: Three Intensity Adjustment | 0~9 | 1 | 7 | I |
| P-73 | Thread pressing actuation angle | Thread pressing actuation angle | 10~150° | 5 | 100 | I |
| P-74 | Thread pressing release angle | Thread pressing release angle | 160~300° | 5 | 270 | I |
| P-75 | Needle position adjustment | Needle position adjustment | 0~240 | 1 | 33 | I |
| P-77 | Setting of beauty sewing | 0：unavailable 1： available | 0/1 | 1 | 0 | I |
| P-78 | Beauty sewing（close sewing）mode setting | 1：Close stitch on start sewing 2：Close stitch on end-sewing 3：Close stitch on both start nad end sewing | 1/2/3 | 1 | 2 | II |
| P-79 | return to factory-set parameter | Special functional parameters 5/8：Restore factory parameters | 0~15 | 1 | 0 | I |
| P-80 | highest speed of sewing | highest speed of sewing | 300~5000(rpm) | 100 | 4000 | II |
| P-81 | Pedal speed percentage | Pedal speed percentage | 50~100 | 1 | 100 | II |
| P-83 | Strength function | Try if the needle can't penetrate the cloth. 0: invalid; 1~15: intensity adjustment | 0~15 | 1 | 0 | II |
| P-84 | Function of cutting line and adding force | Effective when the cutting wire is an electromagnet; 0: invalid; 1~15: intensity adjustment | 0~15 | 1 | 0 | II |
| P-88 | Loose line suction Angle | Loose line suction Angle | 0~360 | 2 | 180 | II |
| P-89 | Loose line release Angle | Loose line release Angle | 0~360 | 2 | 350 | II |
| P-90 | Language selection setting | Setting of language：0：turn off 1：Chinese 2：English | 0~2 | 1 | 1 | II |
| P-91 | Change pitch selection | Change pitch selection 0:Changes Allowed 1:Allowed to change | 0~1 | 1 | 0 | II |
| P-92 | Pedal presser foot lift confirm time | Pedal presser foot lift confirm time | 10~300(ms) | 10 | 80 | II |
| P-93 | The neutral position of the pedal | Trimming the neutral position of the pedal | -15~15(0.1degree) | 1 | 0 | II |
| P-95 | Anti-nest function selection | Anti-nest function selection：0：hread clamp type 1：Anti-next type 2：thread wiper type | 0/1/2 | 1 | 0 | II |
| P-100 | Knee by function setting | Knee by function setting：0：unavailable 1：available | 0/1 | 1 | 1 | II |
| P-101 | The voltage at the zero position of the knee sensor | The voltage at zero position of knee by’s presser foot（Unit:0.01V） | 0~500 | 5 | 270 | II |
| P-102 | Knee sensor’s voltage of maximun travel distance | Knee sendor’s voltage of maximun stravel distance（unit:0.01V） | 0~500 | 5 | 60 | II |
| P-103 | Line tension / loose line function switch | 0 :Line tension control 1 :Ordinary loose line electromagnet control | 0/1 | 1 | 1 | II |
| P-104 | Pattern stop function enable | 0：unavailable 1：Stop the needle need to go through the current number of pattern stitches | 0/1 | 1 | 0 | II |
| P-105 | Presser foot height sensor function setting | Presser foot height sensor function setting 0：unavailable 1： available | 0/1 | 1 | 1 | II |
| P-106 | The voltage adjustment at zero position of presser foot height sensor | The voltage adjustment at zero position of presser foot height sensor（unit:0.01v）（presser foot drop,feed dog is under the table） | 0~250 | 1 | 215 | II |
| P-107 | Overthickness detection sensitivity setting | Presser foot height sensor’s coltage setting of overthickness detection Unit:mv）（Relative to the zero position voltage） | 0~500 | 5 | 100 | II |
| P-109 | Force adjustment of electromagnet for presser foot lifiting | Force adjustment of electromagnet for presser foot lifiting（Cushion when press foot is absorbed） | 50~100 | 1 | 80 | II |
| P-110 | Material thickness and tension control | Increase tension when sewing thick material | 0~10 | 1 | 0 | II |
| P-111 | Overthickness sewing speed | Limit speed of overthickness | 500~3000(rpm) | 50 | 1500 | II |
| P-112 | Overthickness stitch offset coefficient | Overthickness stitch offset deals | 50~150(%) | 1 | 120 | II |
| P-113 | Needle compensate stitch length mode | Needle compensate stitch length mode0：Invaild:vaild（press P114 to set stitch length） | 0/1 | 1 | 0 | I |
| P-114 | Needle compensate stitch length setting | Needle compensate stitch length setting Parameter range of Stitch length 5mm type(1.0mm~5.0mm） Parameter range of stitch length 5mm type(1.0mm~7.0mm） | 10～50（70） | 1 | 35 | I |
| P-115 | Reverse sewing button function setting | 0: reverse sewing 1: tightly sewn 2: reverse sewing 3:reverse sewing+reverse sewing | 0~3 | 1 | 0 | II |
| P-116 | fill needle button function setting | 0: reverse sewing 1: tightly sewn 2: reverse sewing 3:reverse sewing+reverse sewing | 0~3 | 1 | 2 | II |
| P-117 | Dense needle pitch setting | Head button stitch stitch set | 50~150 | 1 | 110 | II |
| P-118 | Number of pretty sewing needle setting | Number of pretty sewing needle setting | 1~10 | 1 | 1 | II |
| P-119 | Pretty sewing needle distance setting | Pretty sewing needle distance setting | 50~150 | 1 | 110 | II |
| P-121 | Start trimming angle of Paragraph 1 | Start trimming angle of Paragraph 1 | 200~300 | 2 | 230 | II |
| P-122 | Paragraph 1 stroke of trimming | Paragraph 1 stroke of trimming | 0~100 | 1 | 40 | II |
| P-123 | Start trimming angle of Paragraph 2 | Start trimming angle of Paragraph 2 | 250~360 | 2 | 330 | II |
| P-124 | Paragraph 2 stroke of trimming | Paragraph 2 stroke of trimming | 0~100 | 1 | 65 | II |
| P-125 | Stop and trimming during fixed-length sewing | 0：invaild 1：vaild | 0/1 | 1 | 0 | II |
| P-126 | loose thread enable when start sewing | 0：invaild 1：vaild | 0/1 | 1 | 1 | II |
| P-127 | Delay before start-sewing loose thread | Delay before start-sewing loose thread | 0~1000 | 10 | 100 | II |
| P-128 | Action time of loose thread when start-sewing | Action time of loose thread when start-sewin | 0~1000 | 10 | 200 | II |
| P-129 | Display backlight brightness Setting | Display backlight brightness Setting | 0~10 | 1 | 5 | I |
| P135 | Midway reverse function selection | 0: unavailable 1: available | 0~1 | 1 | 0 | I |
| P136 | The number of midway stitch set | Pin setting | 1~50 | 1 | 4 | I |
| P137 | Seam midway down the number of times back and forth set | Set times | 1~10 | 1 | 1 | I |
| P-138 | Lock screen function setting | Lock screen function setting：0：invalid 1：valid | 0~1 | 1 | 1 | I |
| P-139 | Lock screen time setting | Lock screen time setting： 0~240 (s) | 0~240 | 1 | 2 | I |
| P-140 | Delay time before thread hooking of Anti-nest | The delay time between the end of trimming and the hook | 0~500ms | 5 | 50 | II |
| P-141 | Thread hooking time of Anti-nest | The action time of the hooked electromagnet | 0~500ms | 5 | 50 | II |
| P-142 | Delay after Anti-nest hook | The delay time of the hook electromagnet turning off | 0~500ms | 5 | 50 | II |
| P-143 | Thread hooking duty ratio of Anti-nest | Adjust the action force of the hooked electromagnet | 0~100 | 1 | 100 | II |
| P-144 | Anti-nest suction work time | The work time of the suction valve | 0~2000ms | 10 | 250 | II |
| P-145 | Thread-straight work time of Anti-nest | The work time of a thread-straight solenoid | 0~500ms | 5 | 50 | II |
| P150 | Maximum stitch length setting | Maximum needle distance setting stitch length 5mm model parameter scale (1.0mm~5.0mm） stitch length 7mm model parameter scale (1.0mm~7.0mm） | 10～50（70）mm | 1 | 50 | II |
| P-151 | First off-line prevented stitch length | 0.invaild 1.vaild | 0~1 | 1 | 0 | II |
| P-152 | First off-line prevented stitch length setting | Stitch length setting Parameter range (1.0mm~5.0mm） | 10~50 | 1 | 40 | II |
| P-153 | Head button inching mode setting | 0：Invalid 1：valid（Press once valid ,press again cancel, such as close sewing, over thickness, presser foot） | 0~1 | 1 | 0 | II |
| P-155 | Needle bar lamp brightness setting | 0：Needle bar light off 1~5：Adjust the brightness of needle bar lamp of 5 gears（HMI control 0/1/2/3/4/5/0switch） | 0~5 | 1 | 5 | II |

Notw：The parameters in the table are standardized.If have any discrepancy,based on reality

4. Error codes

| Error Code | Contents | Possible reasons | Checking and treatment |
| --- | --- | --- | --- |
| E011 E012 | Motor signal error | Motor position sensor signal failure | If electric engine plug is well contacted; if electric engine signal detecting device has been broken; if sewing machine handwheel correctly installed. |
| E021 E023 | Motor overload | motor stall motor overload | If electric engine plug is well contacted; if machine head or thread-cutting mechanism has been blocked completely;f materials are too thick; Electrical signal detection signal whether the normal. |
| E101 | Hardware drivers fault | Current detection abnormal Driving hardware error | Current detection loop system is working properly;Whether the damage to the device driver. |
| E111 E112 | Voltage too high | High input voltage Brake circuit fault Voltage detection error | System into line voltage is too high; Braking resistance are working properly; System voltage detection circuit are working properly. |
| E121 E122 | Voltage too low | Actual low voltage Voltage detection is wrong | If the voltage on the inlet wire is too low Whether the system voltage detection circuit the normal work. |
| E131 | Current circuit fault | Current detection abnormal | Current detection loop system is working properly. |
| E133 | Oz circuit fault | Oz circuit fault | Oz circuit system is working properly. |
| E134 | DBFLT malfunction | Automatic resistance circuit failure | Brake resistor plug is in good contact Brake resistance is damaged |
| E201 | over current | Current detection error | Current detection loop system is working properly Electrical signal is normal. |
| E211 E212 | Abnormal motor operation | Current or voltage detection error | If electric engine plug is well contacted If electric engine signal |
| E301 | Communication error | Sci circuit error | if operation box plug is well contacted; if operation box components are damaged. |
| E302 | Operation inner failure | Sci circuit error | check whether the operating box is damaged. |
| E303 | SPI Communication breakdown | Sci circuit error | check whether the main control board is damaged. |
| E304 | HMI From the main chip communication failure | Sci circuit error | check whether the operating box is damaged. |
| E402 | Pedal ID fault | Pedal verification fault | Pedal connection is loosen. |
| E403 | Pedal zero position fault | The pedal zero position over range | The pedal is damaged or it is not under stop state when correction |
| E501 | Safety switch fault | Safety switch effective | Put down the head or check turned up switch. |
| E502 | Fuel fault alarm | Fuel fault alarm | Add oil Tips |
| E601 | Hardware drivers fault | Feed motor STEP2 hardware overcurrent | Current detection loop system is working properly;Whether the damage to the device driver. |
| E602 | Hardware drivers fault | Feed motor STEP2 software overcurrent | Current detection loop system is working properly;Whether the damage to the device driver |
| E603 | Hardware drivers fault | Feed motor STEP2 Current detection circuit | Current detection loop system is working properly;Whether the damage to the device driver. |
| E604 | Motor signal error | Feed motor STEP2 The initial mechanical angle | Feed motor connector is a good contact |
| E605 | Motor signal error | Feed motor STEP2 at the start, Encoder or rotor stuck | Feed motor connector is a good contact Machinery is stuck |
| E606 | Hardware drivers fault | Feed motor STEP2 Motor winding circuit | Current detection loop system is working properly;Whether the damage to the device driver. |
| E607 | Hardware drivers fault | Presser foot trimming motor STEP1 hardware overcurrent | Current detection loop system is working properly;Whether the damage to the device driver. |
| E608 | Hardware drivers fault | Presser foot trimming motor STEP1 software overcurrent | Current detection loop system is working properly;Whether the damage to the device driver. |
| E609 | Hardware drivers fault | Presser foot trimming motor STEP1 Current detection circuit | Current detection loop system is working properly;Whether the damage to the device driver. |
| E610 | Motor signal error | Presser foot trimming motor STEP1 The initial mechanical angle | Presser foot trimming motor connector is a good contact |
| E611 | Motor signal error | Presser foot trimming motor STEP1 at thestart,Encoder or rotor stuck | Presser foot trimming motor connector is a good contact Machinery is stuck |
| E612 | Hardware drivers fault | Presser foot trimming motor STEP1 Motor winding circuit | Current detection loop system is working properly;Whether the damage to the device driver. |
| E613 | Hardware drivers fault | The feed motor STEP2 is running ,Encoder or rotor stuck | Feed motor connector is a good contact Current detection loop system is working properly;Whether the damage to the device driver. |
| E614 | Hardware drivers fault | The Presser foot trimming motor STEP is running ,Encoder or rotor stuck | Feed motor connector is a good contact Current detection loop system is working properly;Whether the damage to the device driver. |
| P.oFF | Power off Display | Power off | Wait for power supply to resume. |
| EvAL | Trial expired | Trial expired | Contact the dealer processing |
| L.bob | The bottom line tips | The bottom line count value is negative | After replacing the bottom line, press P to cancel the Alert Status |
| P.bob | Remind of counting by piece | The number of counting is 0 | Press S key to enter the interface, and long press "front seam" button for more than 2 seconds to cancel the prompt state |

# 1. Safety instruction

# Please read the operation manual and related sewing machinery datasheet carefully before correct use.
1.1 (1) Power voltage and frequence: please refer to motor and control box nameplate.
(2) Interference from electromagnetic wave:please keep far away strong magnetic or high radiation environment in order
to avoid obstructions and make to misoperation.
(3) Grounding: to avoid the noise obstructions or leakage of electricity accident(inculding sewing machine,motor,control
box and positioner) .
1.2 Please make sure power off at least 1min and then can open control box cover, because there are dangerous high voltage.
1.3 Please turn off the power while repairing or wearing needle in order to protect operater’s safty.

1.4  Used where potential dangers exist.

Used where high voltage and electric danger exist.

1.5Product warranty period of one year on condition that this machine is operated correctly and no man-made damage.

HL989-SG Instructions v1.1