3. System Settings

In unlocked screen, free seam mode, overlapping seam mode, programming seam mode, pattern seam mode, press the key to enter the system setting interface, the system setting interface is as follows:

1. safety precautions

Please read this technical information and the matching sewing machine instructions before use, with the correct use.

1.1(1) Power supply voltage and operating frequency: please follow the specifications indicated by the nameplate of motor and control box.

(2) Electromagnetic wave interference: Please keep away from high frequency magnetic wave machine or radio wave transmitter, etc., so as not to cause the electromagnetic wave to interfere with the wrong action of this drive device.

(3) Grounding: In order to prevent noise interference or leakage accidents, please do a good grounding project (including sewing machines, motors, control boxes, positioners).

1.2 When removing the motor or control box, do not pull in with electricity; there is dangerous high voltage in the control box, so wait more than 1 minute to open the control box cover.

1.3 To ensure personal safety, turn off the power supply during maintenance or needle-piercing operations.

1.4 This symbol indicates that when the machine is installed, if there is an error, it will hurt the human body or the machine will be damaged.

So there will be this sign where the machine is dangerous.

This symbol indicates that there is high voltage and so on, electrical danger will have this sign.

1.5 The warranty period of this device is one year under normal working condition and no human error.

HL989 instructions v2.2

4. fault description

| Fault display | Fault content | Possible causes of failure | Inspection items, processing |
| --- | --- | --- | --- |
| E011 E012 | Motor signal malfunction | Motor position sensor signal malfunction | Electrical plugs are in good contact Damage to motor signal detection devices Sewing machine handwheel installed |
| E021 E023 | Motor overload | Motor blocking Motor overload | Electrical plugs are in good contact Is the nose or thread cutting mechanism stuck Sewing fabric over gauge thickness Current detection signal is normal |
| E101 | Hardware-driven failures | Abnormal current detection Direct drive device | System current detection circuit is working properly Damage to drivers |
| E111 E112 | Over-voltage system | Higher actual voltage Brake circuit fault Error in voltage detection | High incoming voltage Brake resistance works properly The system voltage detection loop works properly |
| E121 E122 | Low voltage system | Lower actual voltage Error in voltage detection | Low incoming voltage The system voltage detection loop works properly |
| E131 | Fault of Current Detection Circuit | Abnormal current detection | System current detection circuit is working properly |
| E133 | OZ circuit fault | Abnormal OZ circuit | Does the system OZ loop work properly |
| E134 | DBFLT fault | Automatic Resistance Circuit Unnormal | Automatic resistance plugs are in good contact Damage to automatic resistance |
| E201 | Excessive motor current | Abnormal current detection Abnormal operation of motor | System current detection circuit is working properly Motor signals are normal |
| E211 E212 | Abnormal operation of motor | Abnormal operation of motor | Electrical plugs are in good contact Motor signal mismatch |
| E301 | Poor operation box communication | Loss of Head Operating Box Communication Data | Operating box plugs are in good contact Damage to operating box devices |
| E302 | Failure E2PROM operating box | Internal failure of operating box | Check that the box device is damaged |
| E303 | SPI communication failures | Internal failure of operating box | Check for damage to main control panel |
| E304 | HMI master-slave chip communication fault | Internal failure of operating box | Check that the box device is damaged |
| E402 | Foot pedal ID malfunction | Pedal Identification Fault | Loose pedal joint |
| E403 | Pedal Zero Correction Fault | Pedal zero correction beyond range | The pedal is not stopped when the pedal is damaged or corrected |
| E501 | Troubleshooting switch | Turnover Switch | Drop the nose or check the flip switch |
| E502 | Oil alarm failure | Oil alarm failure | Refueling tips |
| E601 | Hardware-driven failures | Distribution Motor STEP2 Hardware Overcurrent | System current detection circuit is working properly Damage to drivers |
| E602 | Hardware-driven failures | Distribution Motor STEP2 Software Overcurrent | System current detection circuit is working properly Damage to drivers |
| E603 | Hardware-driven failures | Distribution Motor STEP2 Current Detection Circuit | System current detection circuit is working properly Damage to drivers |
| E604 | Motor signal malfunction | Initial mechanical angle STEP2 feeding motor | Electrical plugs are in good contact |
| E605 | Motor signal malfunction | An initial encoder or rotor STEP2 a feeder motor is jammed | Electrical plugs are in good contact Is the machine stuck |
| E606 | Hardware-driven failures | Electrical machinery STEP2 winding circuit | System current detection circuit is working properly Damage to drivers |
| E607 | Hardware-driven failures | Hardware overcurrent STEP1 voltage pin shearing motor | System current detection circuit is working properly Damage to drivers |
| E608 | Hardware-driven failures | Current STEP1 Software for Voltage Foot Shear Motor | System current detection circuit is working properly Damage to drivers |
| E609 | Hardware-driven failures | STEP1 Current Detection Circuit of Press Foot Shear Motor | System current detection circuit is working properly Damage to drivers |
| E610 | Motor signal malfunction | Initial mechanical angle STEP1 pin shearing motor | Pin shears motor plugs are in good contact |
| E611 | Motor signal malfunction | An initial encoder or rotor stuck STEP1 a pin shearing motor | Pin shears motor plugs are in good contact Is the machine stuck |
| E612 | Hardware-driven failures | Circuit of winding of STEP1 motor for pin shearing motor | System current detection circuit is working properly Damage to drivers |
| E613 | Hardware-driven failures | Encoder or rotor stuck in operation STEP2 feeder motor | Electrical plugs are in good contact System current detection circuit is working properly Damage to drivers |
| E614 | Hardware-driven failures | An encoder or rotor stuck in STEP1 operation of a wire-cutting motor | Pin shears motor plugs are in good contact System current detection circuit is working properly Damage to drivers |
| P.oFF | Power-down display | Power off | Waiting for power to re-open |
| EvAL | Trial protection failures | Trial time | Contact Agent |
| L.bob | Bottom line tips | The bottom line count is negative | After changing the bottom line, press the S button to cancel the prompt |
| P.bob | Piecework Tips | The count value is zero | Press the S key to enter the interface, long press the "front fixed seam" key more than 2 seconds to cancel the state of prompt |

The system parameter table is as follows: click other buttons to enter the corresponding interface

| Debugging mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Number | Name of name | Note |  |  |  |  |
| 1 | Motor Initial Angle Detection | Step forward on the pedal (or press the initial angle of the motor to detect), the electric opportunity rotates a few times, then the LCD screen displays the angle number, the display number represents the installation angle of the motor grating sheet. |  |  |  |  |
| 2 | Pedal zero calibration | Step on the zero calibration button, the system will conduct self-inspection |  |  |  |  |
| 3 | Cutting line action test | Step test |  |  |  |  |
| 4 | Zero reference calibration | Click the zero-bit benchmark button to enter the lower interface. 2 parameter P-6,P-75. appear The P-6 represents the zero pin distance correction value (50~150) and the P-75 represents the head reference position adjustment (0~240).At this interface, the default is 0, adjusting the value of the P-6 parameters so that each needle can be hit in the same position. |  |  |  |  |
| 5 | Calibration of forward and backward stitches | Click on the alignment button to enter the lower interface. Appear three parameters P-7,P-8, needle distance. The P-7 represents the correction value of the stitch distance (50~150(%)), and the P-8 represents the correction value of the stitch distance (50~150(%)). Adjust the value of the P-7,P-8 under different needle spacing so that the actual value of the positive and inverted seam is the set needle spacing value |  |  |  |  |
| System parameters |  |  |  |  |  |  |
| Number | Name of name | Parameter Description | Range of parameters | Default |  | Parameter level |
| Panel settings |  |  |  |  |  |  |
| P-028 | signal mode for turn/lift switch | Setting of signal mode of turn/lift switch of machine head 0：always open 1：always close 2：forbid a protection | 0/1/2 |  | 0 | 1 |
| P-020 | Function selection of machine head button | 0：invalid 1：Manual thread cutting while standby 2：Manual presser foot after trimming | 0/1/2 |  | 0 | 1 |
| P-030 | Baseline count enable | Bottom line counter ;0: unavailable 1：available | 0/1 |  | 0 | 1 |
| P-031 | Baseline initial value setting | Baseline initial value setting | 200~4000(0.1m） |  | 1600 | 1 |
| P-035 | Multipliers | Added value of number of primary shears | 0~50 |  | 0 | 1 |
| P-036 | Number of pieces initial setting | Number of pieces initial setting | 0~1000 |  | 100 | 1 |
| P-038 | Piecework Selection Setting | 0： piece plus 2: piece minus | 0/1 |  | 0 | 1 |
| P-082 | Trial time | trial function ;0: trial function invalid 10~1000: power on trial time | 10~1000 (hours) |  | 0 | 3 |
| P-090 | Language Selection Settings | Language Selection Settings 0: Close 1: Chinese 2: English | 0/1/2 |  | 1 | 2 |
| P-091 | Pin pitch change selection setting (step feeding) | Can I change the selection settings 0: Allow to change 1: Do not allow to change | 0/1 |  | 0 | 2 |
| P-113 | Needle compensate stitch length mode | Needle compensate stitch length mode;0:Changes Allowed 1:Allowed to change | 10~50(70) |  | 50 | 1 |
| P-114 | Pitch setting | Needle distance setting; needle distance 5 mm model parameter range (1 mm ~5 mm) needle distance 7 mm model parameter range (1)7 mm) | 0/1/2/3 |  | 0 | 1 |
| P-115 | Fitting button function setting | Backstitch button function setting ;0: Backstitch 1: Close stitch 2: Fill stitch 3: Backstitch + Fill stitch | 0/1/2/3 |  | 2 | 2 |
| P-116 | Pin button function setting | Pin button function setting ;0: reverse 1: close 2: stitch 3: reverse + stitch | 50~150 |  | 110 | 2 |
| P-117 | Close stitch pitch setting | Head button stitch pitch setting | 0~10 |  | 5 | 2 |
| P-129 | backlight brightness setting | Display backlight setting | 0/1 |  | 0 | 1 |
| P-135 | Selection of Midway Slitting Function | 0： valid 1: invalid | 0/1 |  | 0 | 1 |
| P-138 | Setting of Lock Screen Function | 0： valid 1: invalid | 30~240 (s) |  | 60 | 1 |
| P-139 | Screen time setting | Screen time setting | 0/1 |  | 0 | 2 |
| P-153 | Head button click mode | 0： change the needle distance within the limit angle 1: change the needle distance at any angle (the needle distance may not coincide or break the needle) | 0/1 |  | 0 | 3 |
| P-154 | Type of inverted seam key | Type selection of inverted seam button ;0: General switch 1: stroke switch | 0~5 |  | 5 | 2 |
| P-155 | Brightness adjustment of needle rod lamp | Brightness Adjustment of Iron Rod Lamp | 0~3000 |  | 0 | 3 |
| P-156 | Head refueling reminder time | 0： invalid 15~300 0： power on | 0/1/2 |  | 2 | 2 |
| P-161 | 1/2 patch button seting | 1/2 stitch button seting ;0: Backstitch 1:Close stitch 2: 1/2stitch | 0/1/2 |  | 2 | 2 |
| P-162 | 1/4 patch button setting | 1/4 stitch button setting ;0 Backstitch:1:Close stitch 2:1/4 stitch |  |  |  |  |
| Function setting |  |  | 0/1 |  | 1 | 1 |
| P-010 | Self-made Spacing Counting Mode | When 0： set the number of needles in each section, that is, set the value 1 fixed length sewing to be self-compiled, set the value as the number of patterns, that is, the total number of needles is the number × pattern needles | 0~800ms |  | 40 | 2 |
| P-037 | Timeline | Timeline | 0~9 |  | 7 | 1 |
| P-072 | Adjustment of clamp strength | Adjust clamping force 0: clamping function invalid 1~9: dynamics adjustment | 10~150（degrees） |  | 100 | 1 |
| P-073 | Clamp suction angle | Clamp suction angle | 160~300（degrees） |  | 270 | 1 |
| P-074 | Clamp release angle | Clamp release angle | 0~360（degrees） |  | 180 | 2 |
| P-088 | Loose wire suction angle | Loose wire suction angle | 0~360（degrees） |  | 250 | 2 |
| P-089 | Loose release angle | Loose release angle | 0/1 |  | 1 | 2 |
| P-103 | Line tension/release switching | 0： wire tension control 1: common loose wire electromagnet control | 0/1 |  | 0 | 2 |
| P-104 | Figure - stop function enables | 0： Invalid 1: Need to complete the current pattern needle count | 0~10 |  | 0 | 2 |
| P-110 | Fabric thickness and tension control | Increase tension when sewing thick material | 50~150(%) |  | 120 | 2 |
| P-112 | Compensatory coefficient of over-thick pitch | Overthick pitch compensation | 0/1 |  | 0 | 2 |
| P-125 | Fixed long seam parking shear mode | 0： valid 1: invalid | 1~50 |  | 4 | 1 |
| P-136 | Set the number of stitches | 1~50 needles | 1~10 |  | 1 | 1 |
| P-137 | Back-to-back times | 1~10 times | 10~70 |  | 50 | 2 |
| P-150 | Maximum gauge setting | Maximum needle distance setting; needle distance 5 mm model parameter range (1 mm ~5 mm) needle distance 7 mm model parameter range (1)7 mm) | 0/1 |  | 0 | 2 |
| P-151 | Function switch for seaming protection | 0： valid 1: invalid | 10~50 |  | 40 | 2 |
| P-152 | Setting of needle spacing for seaming anti-stripping | Set parameter range of needle distance (1 mm~5 mm) |  |  |  |  |
| Cutting line setting |  |  | 0~100 |  | 50 | 2 |
| P-017 | duty cycle of shear loose | When the shear line is electromagnet: when the shear line loose line duty cycle (too small will affect the shear line electromagnet suction strength) shear line is step: loose line strength adjustment (thin material) | 100~500(rpm) |  | 300 | 1 |
| P-044 | Cutting speed | Adjust the shear speed | 0~15 |  | 0 | 2 |
| P-084 | Shearing and force function (electromagnet shearing) | Effective when shearing is electromagnet 0: invalid 1~15: force regulation | 150~200 |  | 174 | 3 |
| P-085 | Suction angle of shear line | Cutting line suction angle setting | 200~300 |  | 260 | 3 |
| P-086 | Angle of shear force | Setting of shear line plus angle | 300~360 |  | 340 | 3 |
| P-087 | Cutting line release angle | Setting of release angle of shear line | 200~300 |  | 230 | 2 |
| P-108 | power of trimming | power of trimming | 0~100 |  | 40 | 2 |
| P-121 | The starting angle of the first section of the shear line (step shear line) | The starting angle of the first section of the shear line is set (scissors are effective when stepping) | 250~360 |  | 330 | 2 |
| P-122 | First stroke (step shear) | The first section of the shear line is set (scissors are effective when stepping) | 0~100 |  | 65 | 2 |
| P-123 | The starting angle of the second section of the shear line (step shear line) | The starting angle of the second section of the shear line is set (scissors are effective when stepping) |  |  |  |  |
| P-124 | Second stroke (step shear) | The second section of the shear line opening stroke setting (scissors are effective when stepping) | 0/1 |  | 0 | 2 |
| Bird nest setting |  |  | 1/2/3 |  | 2 | 1 |
| P-077 | Close stitch function settings | 0： invalid 1: valid | 0/1/2 |  | 0 | 2 |
| P-078 | Close stitch mode setting | 1： stitch 2: end stitch 3: end stitch | 1~10 |  | 1 | 2 |
| P-095 | Choice of Bird Nest Function | Bird Nest Function Selection ;0: Clamping Type 1: Bird Nest Type 2: Line Dial | 50~150 |  | 110 | 2 |
| P-118 | Set number of stitches | Set number of stitches | 0/1 |  | 1 | 2 |
| P-119 | Stitch stitch setting | Stitch stitch setting | 0~1000 |  | 100 | 2 |
| P-126 | Sewing loose threads to enable | 0： invalid 1: valid | 0~1000 |  | 200 | 2 |
| P-127 | Delay before opening and loosening | Delay before opening and loosening | 0~500（ms） |  | 50 | 2 |
| P-128 | Action time for seaming | Action time for seaming | 0~500（ms） |  | 50 | 2 |
| P-140 | Delay before bird nest hook | Delay time from end of cut to hook action | 0~500（ms） |  | 50 | 2 |
| P-141 | Action time of bird nest hook | Time of action of hook electromagnet | 0~100 |  | 100 | 2 |
| P-142 | Delay after bird nest hook | Delayed time of turn-off of hook electromagnet | 0~2000（ms） |  | 250 | 2 |
| P-143 | Duty cycle of bird nest hook | Adjust the action of the hook electromagnet | 0~500（ms） |  | 50 | 2 |
| P-144 | Anti-bird nest inspiratory action time | Action time of suction valve |  |  |  |  |
| P-145 | Action time for bird nest alignment | Action time of tappet electromagnet | 0~100 |  | 60 | 2 |
| Press foot setting |  |  | 0~100 |  | 65 | 2 |
| P-012 | Middle pressure foot lift height (step effective) | Set the height of the pressing foot in the middle of sewing | 20~300（rpm） |  | 200 | 2 |
| P -013 | Maximum lifting height (step effective) | Maximum height setting for post-shearing foot | 20~300（rpm） |  | 120 | 2 |
| P-014 | Pressure foot lift speed (step effective) | Step speed of foot lifting | 0~100 |  | 30 | 2 |
| P-015 | Press release speed (step effective) | Step speed released by pressing foot | 0/1 |  | 1 | 1 |
| P-016 | Soft drop duty cycle | Soft drop duty cycle | 0/1/2 |  | 1 | 1 |
| P-024 | Soft press foot drop function | Set the speed to prevent damage to the fabric from slowing down the foot drop | 0/1 |  | 0 | 1 |
| P-025 | Soft foot press | Fetal pressure foot enable setting 0: invalid 1: effective 2: effective after cutting | 10~500（ms） |  | 50 | 2 |
| P-026 | Press foot overthickness function (detection) | Overthickness function setting 0: invalid 1: valid | 0~50（ms） |  | 7 | 2 |
| P-029 | Soft drop time | Press foot soft drop time set, the longer the pressure foot down the slower | 0~10（s） |  | 2 | 2 |
| P-039 | Turn-off time before soft drop | Turn-off time before soft drop | 0~800(ms) |  | 100 | 2 |
| P-040 | Pressure pin loosening function | When 0： lift the foot, the loose line does not move 1~1 0： the tire press foot loose action time | 200~1000（rpm） |  | 500 | 2 |
| P -046 | Foot pressing delay | Delay to confirm that the pin has been lowered | 0~100 |  | 0 | 2 |
| P -047 | Knee speed running | Knee pressure to judge speed | 1~60（s） |  | 25 | 2 |
| P -048 | Running knee height | Lift height of knee support foot | 0~800(ms) |  | 150 | 2 |
| P -049 | Hold time for lifting feet | Force off when lifting foot holds | 0~100 |  | 40 | 2 |
| P -050 | Total pressure output time of lifting foot | Total pressure output time of lifting foot | 0/1 |  | 1 | 2 |
| P -051 | Duty cycle of lift pin output | Duty cycle of lift pin output | 0~500（0.01V） |  | 270 | 2 |
| P -100 | Knee function settings | Knee function setup 0: invalid 1: valid | 0~500（0.01V） |  | 60 | 2 |
| P -101 | Knee Zero Voltage | Zero position voltage value of knee pin | 0/1 |  | 1 | 2 |
| P -102 | Maximum stroke voltage at knee | Maximum stroke voltage value of knee pin | 0~250（0.01V） |  | 215 | 2 |
| P -105 | Foot height function setting | Foot height sensor function settings 0: invalid 1: effective | 0~500（0.01V） |  | 100 | 2 |
| P -106 | Zero adjustment of foot height | Pressure foot height sensor zero voltage adjustment (pressure foot fall, feed teeth under the table board) | 50~100 |  | 80 | 2 |
| P -107 | Sensitivity of over-thickness detection | Overthickness detection pin height sensor voltage setting (relative to zero voltage) |  |  |  |  |
| P -109 | Full pressure duty cycle of pressure foot | Full pressure duty cycle of pressure foot (buffer when pressure foot absorbs) | 500~1500（rpm） |  | 800 | 1 |
| Fixed seam setting |  |  | 0/1 |  | 0 | 1 |
| P -009 | Switches Switch Speed Limit | Speed Limit Speed Speed Speed Rate of Switch | 0/1 |  | 0 | 1 |
| P -018 | Fixation after front joint | After the end of the front fixed seam immediately after stepping on the shear line after the fixed seam function setting 0: invalid 1: effective | 0/1 |  | 0 | 2 |
| P -019 | Stop after the closure | Stop setting after end of front joint ;0: invalid 1: valid | 0/1/2 |  | 0 | 1 |
| P -034 | Standard Solid Seam Speed Mode | Standard Fix Speed Mode Selection ;0: Automatic 1: Pedal Control | 100~3000(rpm) |  | 2200 | 1 |
| P -045 | Switches speed limit | The speed limit treatment of the inverted slit switch can prevent the inverted slit from breaking the needle ;0: no speed limit 1: instantaneous speed limit 2: constant speed limit | 80~120 |  | 100 | 1 |
| P -053 | Front seam velocity | Front seam velocity | 80~120 |  | 100 | 1 |
| P -054 | Repair of front fixing seam | Compensatory coefficient of the front fixed joint | 100~3000(rpm) |  | 2200 | 1 |
| P -055 | Repair of the Back seam of the Front Fixer | Compensating factor of reverse joint of front fixed joint | 80~120 |  | 10 | 1 |
| P -056 | Post-fixation velocity | Post-fixation velocity | 80~120 |  | 100 | 1 |
| P-057 | Correct seam compensation for post-fixing joint | Compensatory coefficient of straight seam of post-fix joint | 100~3000(rpm) |  | 2200 | 1 |
| P-058 | Compensation for Back-up Joint | Compensatory coefficient of back-fix joint | 0/1 |  | 0 | 2 |
| P-059 | Continuous Seam Speed | Continuous Seam Speed |  |  |  |  |
| P-060 | Fixed seam cancel speed limit switch | 0： software has speed limit 1: software does not speed limit (large needle distance high speed needle trace may be unstable) | 100~5000(rpm) |  | 200 | 1 |
| Pedal setting |  |  | 0/1/2 |  | 2 | 1 |
| P-041 | Low speed speed | Pedal minimum speed | 10~50（0.1度） |  | 25 | 2 |
| P-042 | Choice of pedal curve | Pedal speed regulation function adjustment 0: normal 1: acceleration slow 2: acceleration fast | 10~100（0.1度） |  | 50 | 2 |
| P-062 | Start-up trip | The starting position of the pedal relative to the stroke when the pedal is neutral | 10~150（0.1度） |  | 110 | 2 |
| P-063 | Start the acceleration | The position of the pedal starting to accelerate relative to the stroke of the pedal when it is neutral | -100~-10（0.1度） |  | -30 | 2 |
| P-064 | Running high-speed travel | Travel to the highest speed pedal position relative to pedal neutrality | -100~-10（0.1度） |  | -30 | 2 |
| P-065 | The rising stroke of the foot | The pedal position of the foot lift relative to the stroke when the pedal is neutral | -100~-10（0.1度） |  | -60 | 2 |
| P-067 | Cut-line action stroke 1 | When there is no tire pressure foot function, start cutting line pedal position | 10~300(ms) |  | 80 | 2 |
| P-068 | Cut-line action stroke 2 | When the foot is lifted, the pedal position of the starting shear line is relative to the stroke when the pedal is neutral | -15~15 |  | 0 | 2 |
| P-092 | Pedal lift foot command time | Step after the pedal, press the foot command to confirm the time effectively |  |  |  |  |
| P-093 | Pedal neutral position | Pedal neutral trim | 200~5000（rpm） |  | 3700 | 1 |
| Spindle setting |  |  | 0~9 |  | 2 | 1 |
| P-001 | Maximum pedal speed | Set the maximum pedal speed | 200~4000（rpm） |  | 3000 | 1 |
| P-002 | Soft start enable | A few stitches at low speed at the beginning of sewing ;0: close 1~9: number of soft start stitches | 100~3000（rpm） |  | 400 | 1 |
| P-004 | Fixed long seam velocity | Set maximum speed of fixed length seam | 100~3000（rpm） |  | 1000 | 1 |
| P-021 | Soft Startup Speed 1 | Soft Start 1 Pin Speed | 100~3000（rpm） |  | 1500 | 1 |
| P-022 | Soft Startup Speed 1 | Soft Start Needle 2 Speed | 0/1 |  | 0 | 1 |
| P-023 | Soft Startup Speed 1 | Soft Start ~3-9 Pin Speed | 0~240 |  | 175 | 1 |
| P-027 | Power-up location setting | Function setting of machine head automatically finding needle position when power on ;0: invalid 1: effective | 0/1 |  | 0 | 1 |
| P-069 | Lower pin position | Position adjustment of lower stop needle | 0~45（度） |  | 20 | 1 |
| P-070 | Reverse lift function | Reverse lift function after cutting thread ;0: invalid 1: effective | 300~5000（rpm） |  | 4000 | 2 |
| P-071 | Reverse needle angle | Reverse needle angle | 50~100 |  | 100 | 2 |
| P-080 | Maximum sewing speed | Motor maximum speed setting | 0~15 |  | 0 | 2 |
| P-081 | Percentage of pedal speed | Percentage of pedal acceleration | 500~3000（rpm） |  | 1500 | 2 |

Note: the parameters in the table are standardized. If there is any discrepancy, please refer to the actual situation.

2. operation box 明

| Mode Switch key |  | Click switch sewing mode: free seam mode / programming seam mode / pattern seam mode / overlap seam mode. |
| --- | --- | --- |
| Set key |  | Click to enter the system settings interface. |
| Lock screen key |  | Switch lock screen, unlock. Other function keys fail in lock screen state. |
| Front fixing joint |  | Click switch: front fixed seam / front double fixed seam / front four fixed seam / close, and reinforcement needle number / fixed seam pattern number selection |
| Back fixation |  | Click switch: post-fixed / post-double-fixed / post-fixed / closed, and number of strengthened needles / fixed sewing pattern number selection |
| Pattern fixing |  | Short press to open or close pattern fixing function, long press to enter pattern fixing editing page |
| Pin pitch adjustment key |  | Click to adjust the needle spacing. |
| Motor speed |  | Click on the adjustable speed. |
| Piece key |  | Click to enter the piecework interface. |
| Clamp function keys |  | Turn on or off the clamp function. |
| Pinpoint Selection Key |  | Adjust the upper / lower stop needle function. |
| Cut line function keys |  | Turn on or off the shearing function. |
| Close stitch function keys |  | Cycle: front / back / back / back / close. |
| Lift the pin after cutting |  | Turn on or off the pin function after shearing. |
| Soft Start Key |  | Turn on or off the soft start function, the first few stitches will be slower after opening. |
| Auto lift foot key |  | Turn on or off the midway lifting foot function. |
| Bird nest keys |  | Turn on or off the bird nest function. |
| Line tension bond |  | Turn on or off line tension adjustment function. |
| Automatic Sewing Key |  | Turn on or off the automatic seam function. |

Note ：1. Function key icon: black icon indicates open, gray icon indicates close.

2. adjust the value addition and subtraction, click on the function value, pop up the value input interface, input value click OK determine.

3. mode key, click mode switch key, display the corresponding mode name, enter the corresponding mode interface.