5、Failure description

| Error code | Fault content | Reason | Inspection items and how to fix |
| --- | --- | --- | --- |
| E01 | Hardware overcurrent | Current detection is abnorma The motor is running abnormally | Whether the system current detection circuit is working normally Whether the hardware drive device is working normally |
| E02 | Software overcurrent | The motor continues to run with high current | Current detection loop system is working properly |
| E03 | Voltage too high | High input voltage Brake circuit fault Voltage detection error | System into line voltage is too high; Braking resistance are working properly; System voltage detection circuit are working properly |
| E04 | Voltage too low | Actual low voltage Voltage detection iswrong | If the voltage on the inlet wire is too low Whether the system voltage detection circuit the normal work. |
| E05 | Encoder signal is abnormal | Motor sensor signal failure Motor power cable is damaged | Whether the motor encoder interface is in good contact Whether the motor interface is in good contact |
| E06 | Motor signal error | Motor positioning signal failure | Whether the motor encoder interface is in good contactWhether the needle stop substrate is installed in place |
| E07 | Communication error | Sci circuit error | If operation box plug is well contacted; If operation box components are damaged. |
| E08 | Pedal ID fault | Pedal verification fault | Pedal connection is loosen. |
| E09 E19 | motor stall motor overload | motor stall motor overload | If electric engine plug is well contacted; If machine head or thread-cutting mechanism has been blocked completely;f materials are too thick; Electrical signal detection signal whether the normal. |
| E21 | Current circuit fault | Current detection abnormal | Current detection loop system is working properly. |
| E22 | OZ circuit fault | OZ circuit fault | OZ circuit system is working properly. |
| OFF | Power off Display | Power off | Wait for power supply to resume. |

Note: If the above faults cannot be eliminated according to the inspection items, please applicate technical support.

- 1.Safety precautions

- Before use, please read this manual and lockstitch machine's manual at the same time, thanks for your cooperation.

- 1.1 (1) The voltage and working frequency: please according the name plate of motor and contorlling box.

- (2)Electromagnetic interference:Please stay away from high-frequency magnetic wave machines or Radio wave transmitter etc.In order to prevent the generated electromagnetic waves from interfering with the drive device and causing wrong action

- (3)Grounded:To prevent noise interference or leakage accidents,Please do grounding works including sewing machine, motor and control box.

- 1.2 If Disassemble motor and controlling box,please Turn off the power in advance.Dangerous high voltage electricity in the electric control box, so please waiting more than 1 mins after Turn off the power, then to disassemble the control box

- 1.3 To ensure the safety of life,Please turn off the power when repair the machine or threading needles.

- 1.4  This symbol indicates when the machine is installed,if made a mistake,the human body or the machine may be damaged,Therefore, there will be this mark in the dangerous place of the machine.

- This symbol indicates that there is high voltage, etc.Electrically dangerous places have this mark.

- 1.5 This products guarantees a one-year warranty under normal working conditions and no-one error operation

Install Instruction（Photo in the right side）

- 2.1、Belt Install Instruction：

- Make sure the sewing machine hand wheel and motor pulley in the same level; and control the tightness of belt(use two fingers hold belt tight, two belts cannot be touched and not too tight);

- 2.2、Connecting Rod install instruction：

- Make sure the perpendicularity of the rod, or it will affect the life and performance of the pedal.

2.Install Requirement

| Function parameter edit key |  | Enter or exit the editing of function parameters |
| --- | --- | --- |
| Parameter view save key |  | View and save the content of the selected parameter number: After selecting the parameter number, press this key to view and modify the operation, after modifying the parameter value, press this key to exit and save the parameter |
| Up selection key |  | - 1.Increment key for parameter item in parameter selection area - 2. Set the value increment key in the parameter content area |
| Down Select key |  | - 1.Decrease key of parameter item in parameter selection area - 2.Set the value decreasing key in the parameter content area |
| Slow start key |  | - 1.To setting open or cancel the slow start sewing function - 2.If this key and the‘down selection key’are combination to one key, press and hold for 1 second to turn on or cancel the slow start sewing function |
| Stop needle position selection key |  | - 1.Switch the stop position of the needle after sewing - (upper position/down position) - 2.If this key and the‘up selection key’are combination to one key, press it for 1 second to switch the needle position |

3、Key Description

6、Parameter list

| Parameter | Description | Range | Initial value | Content value name description and remarks |
| --- | --- | --- | --- | --- |
| P01 | Maximum speed | 200-6500 | 3700 | Maximum speed setting during sewing |
| P02 | Sychronizer function selection | 0-1 | 1 | "0" without signal,"1” mean without signal |
| P03 | Sychronizer selection | 0-1 | 0 | "0" mean Needle up,"1" mean Needle down |
| P04 | Motor running direction | 0-1 | 0 | "0": for interlock, "1" : for overlock |
| P06 | Soft start switch | 0-1 | 0 | "0" mean open, "1" mean turn off |
| P07 | Soft start Speed | 200-1500 | 400 | Soft start speed function setting |
| P08 | Number of soft start stitchs | 1~15 | 1 | Number of soft start stitchs |
| P09 | Minimum speed | 200-500 | 200 | Minimum speed setting during sewing |
| P10 | Upper needle position adjustment | 0-100 | 0 | Upper needle position adjustment |
| P11 | Down needle position adjustment | 0-100 | 10 | Position adjustment of lower needle stop |
| P12 | Acceleration curve adjustment | 0-4 | 1 | Speed controller acceleration and climb slope setting |
| P13 | Recovery settings | 0-2 | 0 | 0：not recover；1：Recovery common parametric; 2：all Recovery parametric |
| P14 | Trinning function selection | 0-2 | 2 | "0": off, "1" : on,"2" : Step back output signal |
| P15 | Trimming action time | 0-100 | 2 | Unit: 0.1s |
| P16 | Trimming delay time | 0-100 | 0 | Unit: 0.1s |
| P18 | Suction function selection | 0-2 | 2 | "0": off, "1" : step back and turn on,"2" : Normally open |
| P19 | Suction action time | 0-100 | 2 | Unit: 0.1s |
| P20 | Suction delay time | 0-100 | 0 | Unit: 0.1s |

4.Instructions

4.1 Monitoring mode

Press the S key for 2 seconds to enter the monitoring state, press ▲▼ to change the monitoring serial number, the monitoring value cannot be modified. The specific monitoring interface is shown in the table below.

4.2 Quick operation mode

1.In normal power-on state, press P+slow start sewing key to switch between forward and reverse

2.In normal power-on state, press P+ up and down needle position keys to switch with or without needle stop position.

| Display No: | Item number | unit |  | Display No: | Item number | unit |
| --- | --- | --- | --- | --- | --- | --- |
| SPd | Speed | 10rpm |  | HALL | HALL | / |
| CUr | Current | 0.1A |  | id | model | / |
| UdC | Voltage | V |  | vEr | version | / |
| PdL | Pedal | / |  |  |  |  |